| Jefferson Lab Alignment Group Data Transmittal |  |  |  |
| --- | --- | --- | --- |
| TO: J. LeRose |  | DATE: Dec 16 2003 |  |
| FROM: Chris Curtis | Checked: |  | # : A913 |

DETAILS:

Below are the results from the electron (left) and hadron (right) spectrometer survey carried out on December 3rd and 4th, 2003. Also included are the results from the superharp survey carried out on the 3rd. The values represent the difference in millimeters from the ideal position of each fiducial. A positive X value is to the beam left, a positive Y is too high.

================ RESULTS =============== E120303A

The central ray of the spectrometer is at  -12.492 degrees.

It is missing the defined target center by         1.84 mm   upstream,

and   -0.60 mm vertically (positive = up).

If the offset is corrected by secondary alignment, the

spectrometer will be at  -12.504 degrees.

*.9PR : 0.106		*.3DD : 1.06

================ RESULTS =============== H120403A

The central ray of the spectrometer is at   12.512 degrees.

It is missing the defined target center by         0.93 mm downstream,

and   -0.50 mm vertically (positive = up).

If the offset is corrected by secondary alignment, the

spectrometer will be at   12.506 degrees.

*9PR : 0.132		*.3DD : 1.47

Superharps

DX	          DY		locations in mm (+ is beam left)

SH1H01A        -0.01        -0.18

SH1H01B         0.05        -0.18

SH1H01C         0.06        -0.15

SH1H02A        -0.31        -0.07

SH1H02B        -0.24        -0.11

SH1H02C        -0.25        -0.12