Hall A “LEDEX” RunPlan

1st Low-Energy Beam Period (Eo = 362 MeV): July 24 – Aug. 20, 2006

E05-103

PROCEDURES FOR PHOTODISINTEGRATION PRODUCTION SEQUENCE

1. Change Spectrometer Settings and Take Pointing Measurements

- Change HRS-L momentum & angle settings to those specified for the current PRODUCTION Kinematics Point from the Kin-Table (follow Counting House “Whiteboard” instructions from J. Lerose for cycling quads when setting momentum; see “How To” on LEDEX webpage).

- Small collimator on HRS-L (will be already bolted in place).

- Check beam position on BPMs (few-tenths of mm on each); 
set Raster ON: ASK MCC for “6 x 4.5” (  3 mm in X and Y)

- Set prescales T3=low; other prescales set high (65535) for low deadtime.

- Beam current of a few microamps  (up to max DAQ rate of 2-3 kHz)

- Set target to following settings, and take 5 minutes of pointing data at each setting (separate run for each target). (Target C “optics” is not the slanted one)

| ph (GeV/c) | h (deg) | Target | Time (min) |
| --- | --- | --- | --- |
| Look up for current Kin. Point in Kin-Table |  | C “optics” | 5 |
|  |  | 4 cm Dummy | 5 |
|  |  | 15 cm Dummy | 5 |

2. Production Measurement Sequence:

- FPP Carbon Doors: place either the 3” doors (if momentum above ~660 MeV/c), 1.5” doors (if central momentum between ~560-660 MeV/c) or NO doors (less than ~560 MeV/c)  check FPP Figure of Merit Simulation /  runplan document.

- Check beam position on BPMs; ensure RASTER ON (see notes above).

- Set prescales T3=low; other prescales set high (65535) for low deadtime.

- Pre-run estimates are that Beam Currents between 1-4 A will  provide the max DAQ rate of between 2-3 kHz.  In each case, the current should be adjusted to give max DAQ rate between 2-3 kHz.

- Set target/radiator to settings indicated in Table below, but:

  - FIRST: TAKE A FEW MINUTES OF DATA AT EACH TARGET/RADIATOR COMBINATION IN TABLE BELOW IN ORDER TO DETERMINE RATES (AND MAX CURRENT THAT CAN BE USED FOR EACH COMBO) – THEN STEFFEN’S LEXEX WEB-CALCULATOR WILL BE USED TO DETERMINE MODIFIED RUN TIMES FOR EACH RUN TYPE (the split of times given below is a rule-of-thumb split of luminosity into 50%-25%-25%-short).

  - THEN: take FPP data for the approximate times indicated below (final run times will come from above running of Steffen’s calculator, using rates/currents for each point).

| ph (GeV/c) | h (deg) | Target | Radiator | DAQ Rate (kHz) | Time (hr) |
| --- | --- | --- | --- | --- | --- |
| As set in first step above (1.) |  | LD2 | IN | 2-3 (max) | ~6 |
|  |  | LD2 | NONE | 2-3 (max) | ~3 |
|  |  | LH2 | IN | 2-3 (max) | ~3 |
|  |  | LH2 | NONE | 2-3 (max) | 0.2 |