|  | 3210Appendix T2 Task Hazard Analysis (THA) Worksheet |  |
| --- | --- | --- |

| Author: |  | Ed Folts |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | 9/21/11 |  |  |  |  | Task #: If applicable |  |  |  |  | Frequency of use: |  |  | As necessary |
| Complete all information. Use as many sheets as necessary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Task Location: |  |  |  | Hall a |  |  |  |  | Task Title: |  |  | Removal of G2P beam dump insert |  |  |
| Division: |  |  | Physics |  |  |  |  |  |  | Department: |  |  | Hall a |  |
| Lead Worker: |  |  |  |  | Ed Folts |  |  |  |  |  |  |  |  |  |
| Mitigation already in place: Standard Protecting Measures Work Control Documents |  |  |  |  |  |  | Radiation training |  |  |  |  |  |  |  |

| Sequence of Task Steps | Task Steps/Potential Hazards | Consequence Level | Probability Level | Risk Code (before mitigation) | Proposed Mitigation (Required for Risk Code >2) | Safety Procedures/ Practices/Controls/Training | Risk Code (after mitigation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Radiation exposure | H | M | 4 | BUILT IN SHIEDLING, ACCESS UNDER RADCON CONTROL,BODY SHIELDING. SUPLEMENTAL DOSEMITRY | RAD II TRAINING REQUIRES FOR BALCONY ACCESS, ACCESS TO BALCONY IS UNDER RADCON CONTROL, DUMP SHIELDING IS INSTALLED, BODY SHIELDING IS AVAILABLE IF REQUIRED BY RADCON | 2 |
|  | Magnetic field | H | M | 4 | MAGNETS OFF AND INTERLOCKED TO GATES | NO PACMEKERS. MAGNETS OFF AS PER PROCEDURE AND OFF DUE TO INTERLOCKS WHEN ACCESS GATES ARE OPEN | 1 |
|  | Thin vacuum window | M | M | 3 | WINDOW GUARDS | GUARDS INSTALL AT START OF PROCEDURE | 1 |
|  | Heavy weight | M | M | 3 | INSERTION/DESERTION FIXTURE | INSERTS WILL BE MOVES USING FIXTURE UNDER UPPER TARGET BALCONY SUPPORT | 1 |
|  | Loud noise | M | L | 2 | EAR PROTECTION |  | 1 |
|  | Lead exposure | L | L | 2 | Paint, gloves |  | 1 |
|  | Congested work area | L | L | 1 |  |  | 1 |

|  | Highest Risk Code before Mitigation: | 4 |  | Highest Risk Code after Mitigation: | 2 |
| --- | --- | --- | --- | --- | --- |

When completed, if the analysis indicates that the Risk Code before mitigation for any steps is “medium” or higher (RC≥3), then a formal Work Control Document (WCD) is developed for the task.  Attach this completed Task Hazard Analysis Worksheet.  Have the package reviewed and approved prior to beginning work.  (See ES&H Manual Chapter 3310 Operational Safety Procedure Program.)

|  | ISSUING AUTHORITY | FORM TECHNICAL POINT-OF-CONTACT | APPROVAL DATE | EFFECTIVE DATE | EXPIRATION DATE | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ESH&Q Division | Harry Fanning | 10/05/09 | 01/01/09 | 10/05/12 | 0 |  |  |

This document is controlled as an on line file.  It may be printed but the print copy is not a controlled document.  It is the user’s responsibility to ensure that the document is the same revision as the current on line file.  This copy was printed on 7/29/2026.