Jefferson Lab Alignment Group

DATA TRANSMITTAL

TO:	Jacques Marroncle						DATE: 11 Mar 1999

FROM:    Chris Curtis	                       			Checked:		# :  502

DETAILS:

Below are the results of the recent survey performed on the e-p scatter chamber     scintillators. Coordinates are given in millimeters in the e-p chamber system generated for the tooling balls inside the chamber. This system has +X along the beamline and +Y downwards. The scintillator locations are indicated on the accompanying sketch.

Point		     X		   Z		      Y

SC10      	  334.7     	119.9   	  -572.0

SC15      	  336.4     	   -1.5     	   579.6

SC16     	1501.3     	125.2    	 2596.0

SC17     	1495.8     	   -2.8    	 2601.3

SC18     	1457.5     	   -0.2   	-2522.7

SC21L     	  814.3     	494.8   	   656.5

SC21R     	  812.5     	255.7   	   656.7

SC22L     	  874.7     	489.3   	   519.4

SC22R     	  872.9     	247.3   	   519.9

SC23L     	  923.5     	494.2   	   412.1

SC23R     	  922.8     	253.1   	   412.7

SC24L     	  955.5     	490.9   	   337.3

SC24R     	  954.9     	250.0   	   337.8

SC25L     	  985.3     	492.7   	   269.2

SC25R     	  985.0     	250.5   	   269.9

SC26L    	1011.3     	494.4   	   215.7

SC26R    	1010.0     	256.0   	   216.2

SC27L    	1034.1     	491.2   	   168.1

SC27R    	1033.3     	251.8   	   168.5

SC28L    	1034.0     	493.1   	  -166.1

SC28R    	1033.9     	257.0   	  -166.0

SC29L    	1012.5     	497.0   	  -213.5

SC29R    	1012.8     	258.8   	  -213.8

SC30L     	  987.0     	494.6   	  -268.7

SC30R     	  987.0     	253.8   	  -268.6

SC31L     	  958.9     	492.8   	  -335.7

SC31R     	  958.7     	255.9   	  -336.0

SC32L     	  924.9     	496.3   	  -410.3

SC32R     	  924.9     	252.9   	  -410.6

SC33L     	  876.5     	492.9   	  -518.3

SC33R     	  876.1     	249.3   	  -519.3

SC34L     	  813.9     	491.6   	  -654.8

SC34R     	  813.8     	256.8   	  -655.5

DATA TRANSMITTAL (cont.)

# : 502						DATE:  11 Mar 99 		PAGE : 2 of 2

SC18

SC10

SC34

33

32

31

30

29

28

27

26

25

24

23

22

SC21			SC15

SC16 / SC17