Jefferson Lab Alignment Group

DATA TRANSMITTAL

TO:	Javier Gomez, John LeRose				DATE: 3 May 1999

FROM:    Chris Curtis						Checked:	       	 # 520

DETAILS:

Below are the results from the spectrometer pointing survey carried out on April 28th. The survey was made of the electron spectrometer concurrently with a superharp survey (results in DTM #518) at the following angle.

E042899A	 12.5 degrees

================ RESULTS =============== E042899A

The central ray of the spectrometer is at  -12.527 degrees.

It is missing the defined target center by    .95 mm   upstream,

and    -.41 mm vertically (positive = up).

If the offset is corrected by secondary alignment, the

spectrometer will be at  -12.533 degrees.

To achieve this optimal setting,

make the following adjustments:

Horizontal corrections:

Move rear jacks along tangent     .97 mm   upstream.

Vertical corrections (positive = up):

Central jack:            1.63 mm.

Upstream rear jack:      2.59 mm.

Downstream rear jack:    3.16 mm.

*.3DD = 1.36

*.9PR = 0.12