Jefferson Lab Alignment Group

DATA TRANSMITTAL

TO: J. Arrington, J. Gomez, N. Liyanage, J. LeRose      	DATE: Apr 24, 2002

FROM:  Chris Curtis                               				Checked:		# : A761

DETAILS:

Below are the results from the spectrometer pointing surveys carried out on the 	Electron (left) arm and Hadron (right) arm on April 19th & 23rd, 2002.

================ RESULTS =============== E041902A

The central ray of the spectrometer is at  -21.486 degrees.

It is missing the defined target center by   2.19 mm   upstream,

and    0.58 mm vertically (positive = up).

If the offset is corrected by secondary alignment, the

spectrometer will be at  -21.501 degrees.

*.9PR : 0.130	 	*.3DD : 1.16

================ RESULTS =============== H041902A

The central ray of the spectrometer is at   44.455 degrees.

It is missing the defined target center by   0.32 mm   upstream,

and    0.80 mm vertically (positive = up).

If the offset is corrected by secondary alignment, the

spectrometer will be at   44.458 degrees.

9PR : 0.098		*.3DD : 1.26

================ RESULTS =============== E042302A

The central ray of the spectrometer is at  -22.147 degrees.

It is missing the defined target center by   2.45 mm   upstream,

and    0.72 mm vertically (positive = up).

If the offset is corrected by secondary alignment, the

spectrometer will be at  -22.163 degrees.

*.9PR : 0.126	 	*.3DD : 1.18