| Jefferson Lab Alignment Group Data Transmittal |  |  |  |
| --- | --- | --- | --- |
| TO: J. LeRose, P. Bertin, D. Meekins |  | DATE: Sep 15 2004 |  |
| FROM: C. Gould, J. Dahlberg | Checked: |  | # : A950 |

DETAILS:

Below are the results from the Sept 3, 2004 survey performed on the Hall A target ladder.  The values are relative to ideal target position (in millimeters), with +X to the beam left, +Z downstream, and +Y above.  The repeated measurement when target cell 1 was moved vertically back into beam center was on one fiducial point and represents a relative change for that fiducial only.

LOCATION			   Z		   X		  Y

Tgt. cell 1 (top)			-0.36		-1.67		 0.13

Tgt. cell 2			-0.10		-0.98		 0.07

Tgt. cell 3			-0.01		-0.94		-0.28

Tgt. cell 4 (bot)			 0.03		-0.59		-0.40

Tgt. cell 1 (repeat)		-0.22		 0.21		 0.07

Center foil (optics tgt.)	-0.91

4 cm upst		         -19.91		-1.45		 4.16

dnst		          20.04		-2.94		-1.73

15 cm upst		         -74.18		-2.43		-0.97

dnst		          75.73		-4.75		-2.38

Crosshair tgt.			-0.53		-1.90		-0.29

Solid tgt ladder Pitch:	-0.1261 deg. (ccw looking from beam right).

Yaw:	 0.6074 deg. (ccw looking from above).