Jefferson Lab Alignment Group

DATA TRANSMITTAL

TO:	Jack Segal							DATE: 30 Mar 1999

FROM:    Jim Dahlberg	                      			Checked:		# :  510

DETAILS:

Below are the results of the recent survey performed on the Hall A VDC chambers.

The values are in millimeters and represent the relative movement of each chamber

after the supports were changed out.  A (+Y) value indicates a rise, and a (–X) value

indicates a shift to the beam right.

LOCATION				D Z		D X		D Y

HADRON

Upstream beam left			0.0		 -0.05		+0.02

Upstream beam right		0.0		 -0.05		+0.04

Downstream beam left		0.0		+0.20		 -0.02

Downstream beam right		0.0		+0.20		+0.05

ELECTRON

Upstream beam left			0.0		-0.22		+0.08

Upstream beam right		0.0		-0.22		+0.18

Downstream beam left		0.0		-0.17		 -0.02

Downstream beam right		0.0		-0.17		+0.17