MATERIAL SAFETY      3M

DATA SHEET           3M Center

St. Paul, Minnesota

55144-1000

1-800-364-3577 or (651) 737-6501 (24 hours)

Copyright, 1999, Minnesota Mining and Manufacturing Company.

All rights reserved.  Copying and/or downloading of this

information for the purpose of properly utilizing 3M products

is allowed provided that:

1) the information is copied in full with no changes unless

prior agreement is obtained from 3M, and

2) neither the copy nor the original is resold or otherwise

distributed with the intention of earning a profit thereon.

DIVISION:  3M SPECIALTY MATERIALS

TRADE NAME:

PFG-5040 3M (TM) SPECIALTY GAS

ID NUMBER/U.P.C.:

98-0211-8851-5    -     -     -   98-0211-8852-3    -     -     -

98-0211-8853-1  00-51135-10966-3  98-0211-8854-9  00-51135-10967-0

98-0211-8892-9  00-51135-11000-3

ISSUED: December 13, 1999

SUPERSEDES: September 29, 1998

DOCUMENT: 07-3459-0

-----------------------------------------------------------------------------

1. INGREDIENT                            C.A.S. NO.         PERCENT

-----------------------------------------------------------------------------

Perfluorobutane (C4F10).................   355-25-9              > 99.5

The components of this product are in compliance with the chemical

notification requirements of TSCA.  All applicable chemical

ingredients in this material are listed on the European Inventory of

Existing Chemical Substances (EINECS), or are exempt polymers whose

monomers are listed on EINECS.

-----------------------------------------------------------------------------

2. PHYSICAL DATA

-----------------------------------------------------------------------------

BOILING POINT:................. ca. -2 C

Typical

VAPOR PRESSURE:................ ca. 1680 mmHg

@ 20 C

VAPOR DENSITY:................. ca. 8.4 Air=1

EVAPORATION RATE:.............. > 1 BuOAc=1

SOLUBILITY IN WATER:........... neglig.

SPECIFIC GRAVITY:.............. ca. 1.5 Water=1

PERCENT VOLATILE:.............. 100 %

pH:............................ N/A

VISCOSITY:..................... < 10 centipoise

MELTING POINT:................. N/D

-----------------------------------------------------------------------------

Abbreviations: N/D - Not Determined  N/A - Not Applicable  CA - Approximately

MSDS: PFG-5040 3M (TM) SPECIALTY GAS

December 13, 1999                                                     PAGE  2

-----------------------------------------------------------------------------

2. PHYSICAL DATA     (continued)

-----------------------------------------------------------------------------

APPEARANCE AND ODOR:

Liquefied Gas

-----------------------------------------------------------------------------

3. FIRE AND EXPLOSION HAZARD DATA

-----------------------------------------------------------------------------

FLASH POINT:................... None

FLAMMABLE LIMITS - LEL:........ Nonflammable

FLAMMABLE LIMITS - UEL:........ Nonflammable

AUTOIGNITION TEMPERATURE:...... Nonflammable

EXTINGUISHING MEDIA:

Nonflammable.

SPECIAL FIRE FIGHTING PROCEDURES:

When fire fighting conditions are severe and total thermal

decomposition of the product is possible, wear full protective

clothing, including helmet, self-contained, positive pressure or

pressure demand breathing apparatus, bunker coat and pants, bands

around arms, waist and legs, face mask, and protective covering for

exposed areas of the head.  No unusual effects are anticipated during

fire extinguishing operations. Avoid breathing the products and

substances that may result from the thermal decomposition of the

product or the other substances in the fire zone. Keep containers

cool with water spray when exposed to fire to avoid rupture.

UNUSUAL FIRE AND EXPLOSION HAZARDS:

Exposure to extreme heat can give rise to thermal decomposition. See

section 8, Health Hazard Data.

NFPA HAZARD CODES:  HEALTH: 1  FIRE: 0  REACTIVITY: 0

UNUSUAL REACTION HAZARD: none

-----------------------------------------------------------------------------

4. REACTIVITY DATA

-----------------------------------------------------------------------------

STABILITY: Stable

INCOMPATIBILITY - MATERIALS/CONDITIONS TO AVOID:

Finely divided active metals, Alkali and alkaline earth metals.

HAZARDOUS POLYMERIZATION: Hazardous polymerization will not occur.

HAZARDOUS DECOMPOSITION PRODUCTS:

See Health Hazard Data section.

-----------------------------------------------------------------------------

Abbreviations: N/D - Not Determined  N/A - Not Applicable  CA - Approximately

MSDS: PFG-5040 3M (TM) SPECIALTY GAS

December 13, 1999                                                     PAGE  3

-----------------------------------------------------------------------------

5. ENVIRONMENTAL INFORMATION

-----------------------------------------------------------------------------

SPILL RESPONSE:

Observe precautions from other sections.  Ventilate area.  Close

cylinder or if necessary vent in hood or remote area.  Place leaking

containers in a well-ventilated area, preferrably in an exhaust hood,

if available, or outdoors.

RECOMMENDED DISPOSAL:

Incinerate at a facility equipped to handle gaseous waste.  Facility

must be capable of handling aerosol cans.  Combustion products will

include HF.  Dispose of empty cans in a sanitary landfill.  Reclaim

if feasible.

ENVIRONMENTAL DATA:

No data available.

REGULATORY INFORMATION:

Volatile Organic Compounds: N/A.

VOC Less H2O & Exempt Solvents: N/A.

Since regulations vary, consult applicable regulations or authorities

before disposal.  U.S. Clean Water Act, Section 307, Toxic Pollutants

= None.  U.S. EPA Hazardous Waste Number =  None (Not U.S. EPA

Hazardous).

Components are in compliance with TSCA, EINECS, and MITI.

Transportation Information:

Regulated for ground, air and ocean transport (UN3163).

OTHER ENVIRONMENTAL INFORMATION:

EPCRA 311/312 Reportable Quantity = Not Reportable.

EPCRA HAZARD CLASS:

FIRE HAZARD: No  PRESSURE: Yes  REACTIVITY: No  ACUTE: No  CHRONIC: No

-----------------------------------------------------------------------------

6. SUGGESTED FIRST AID

-----------------------------------------------------------------------------

EYE CONTACT:

Immediately flush eyes with large amounts of water. Get immediate

medical attention.

SKIN CONTACT:

No need for first aid is anticipated.

INHALATION:

Remove person to fresh air. If not breathing, give artificial

respiration. If breathing is difficult, get immediate medical

attention.

-----------------------------------------------------------------------------

Abbreviations: N/D - Not Determined  N/A - Not Applicable  CA - Approximately

MSDS: PFG-5040 3M (TM) SPECIALTY GAS

December 13, 1999                                                     PAGE  4

-----------------------------------------------------------------------------

6. SUGGESTED FIRST AID     (continued)

-----------------------------------------------------------------------------

IF SWALLOWED:

No need for first aid is anticipated.

OTHER FIRST AID INFORMATION:

NOTE TO PHYSICIANS: Exposure to high concentration may increase

"myocardial irritability." Do not administer sympathomimetic drugs

(i.e. adrenaline) unless absolutely necessary. No specific antidote.

Supportive care and treatment based on the judgement of physician in

response to the patient are recommended.

-----------------------------------------------------------------------------

7. PRECAUTIONARY INFORMATION

-----------------------------------------------------------------------------

EYE PROTECTION:

Wear safety glasses with side shields.

SKIN PROTECTION:

Avoid skin contact.  Wear appropriate gloves when handling this

material.

RECOMMENDED VENTILATION:

Use with adequate dilution ventilation.

RESPIRATORY PROTECTION:

Avoid prolonged breathing of vapors.

PREVENTION OF ACCIDENTAL INGESTION:

Not applicable.

RECOMMENDED STORAGE:

Store in a cool place.  Keep container closed when not in use.  Store

in explosion-proof containers.

FIRE AND EXPLOSION AVOIDANCE:

Nonflammable.

Nonflammable.

OTHER PRECAUTIONARY INFORMATION:

No smoking: Smoking while using this product can result in

contamination of the tobacco and/or smoke and lead to the formation

of the hazardous decomposition products mentioned in the Reactivity

Data section of this MSDS.

HMIS HAZARD RATINGS:  HEALTH: 0  FLAMMABILITY: 0  REACTIVITY: 0

PERSONAL PROTECTION: X (See precautions, section 7.)

-----------------------------------------------------------------------------

Abbreviations: N/D - Not Determined  N/A - Not Applicable  CA - Approximately

MSDS: PFG-5040 3M (TM) SPECIALTY GAS

December 13, 1999                                                     PAGE  5

-----------------------------------------------------------------------------

7. PRECAUTIONARY INFORMATION     (continued)

-----------------------------------------------------------------------------

EXPOSURE LIMITS

INGREDIENT                               VALUE  UNIT       TYPE   AUTH  SKIN*

-----------------------------------------------------------------------------

Perfluorobutane (C4F10)..............    NONE   NONE       NONE   NONE

* SKIN NOTATION:  Listed substances indicated with 'Y' under SKIN refer to

the potential contribution to the overall exposure by the cutaneous route

including mucous membrane and eye, either by airborne or, more particularly,

by direct contact with the substance.  Vehicles can alter skin absorption.

SOURCE OF EXPOSURE LIMIT DATA:

- NONE:  None Established

-----------------------------------------------------------------------------

8. HEALTH HAZARD DATA

-----------------------------------------------------------------------------

EYE CONTACT:

Eye contact is not expected to occur during normal use of the

product.

Frostbite: signs/symptoms can include pain, clouding of the cornea,

redness, swelling and blindness.

SKIN CONTACT:

Frostbite: signs/symptoms can include firm blanched areas, redness,

pain, tissue destruction, swelling and scar tissue formation.

INHALATION:

Simple Asphyxiation: signs/symptoms can include light-headedness,

rapid heart beat, a feeling of suffocation and blue color.

Unconsciousness and death may occur if oxygen deprivation is severe

or prolonged. Simple asphyxiation results from the displacement of

oxygen in air to a concentration which is less than that necessary to

maintain life.

IF SWALLOWED:

Ingestion is not a likely route of exposure to this product.

No information was found regarding effects from swallowing.

OTHER HEALTH HAZARD INFORMATION:

If the product is exposed to extreme conditions of heat from misuse

or equipment failure, toxic decomposition products that include

hydrogen fluoride can occur.

Hydrogen fluoride has an ACGIH threshold limit value of 3 parts per

million parts of air, ppm, of fluoride as a ceiling limit and an OSHA

PEL of 3 ppm of fluoride as an eight hour time-weighted average and

-----------------------------------------------------------------------------

Abbreviations: N/D - Not Determined  N/A - Not Applicable  CA - Approximately

MSDS: PFG-5040 3M (TM) SPECIALTY GAS

December 13, 1999                                                     PAGE  6

-----------------------------------------------------------------------------

8. HEALTH HAZARD DATA     (continued)

-----------------------------------------------------------------------------

6 ppm of fluoride as a Short Term Exposure Limit.

A 3M Product Toxicity Summary Sheet is Available.

EXPOSURE ROUTE               SUMMARY OF TOXICITY FINDINGS

Oral                         Practically Non-Toxic

Inhalation                   Simple Asphyxiant

Not a cardiac sensitizer where tested.

-----------------------------------------------------------------------------

SECTION CHANGE DATES

-----------------------------------------------------------------------------

PRECAUTIONARY INFO. SECTION CHANGED SINCE September 29, 1998 ISSUE

-----------------------------------------------------------------------------

Abbreviations: N/D - Not Determined  N/A - Not Applicable  CA - Approximately

-----------------------------------------------------------------------------

The information in this Material Safety Data Sheet (MSDS) is believed to

be correct as of the date issued.  3M MAKES NO WARRANTIES, EXPRESSED OR

IMPLIED, INCLUDING, BUT NOT LIMITED TO, ANY IMPLIED WARRANTY OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE OR COURSE OF

PERFORMANCE OR USAGE OF TRADE.  User is responsible for determining

whether the 3M product is fit for a particular purpose and suitable for

user's method of use or application.  Given the variety of factors that

can affect the use and application of a 3M product, some of which are

uniquely within the user's knowledge and control, it is essential that

the user evaluate the 3M product to determine whether it is fit for a

particular purpose and suitable for user's method of use or application.

3M provides information in electronic form as a service to its customers.

Due to the remote possibility that electronic transfer may have resulted

in errors, omissions or alterations in this information, 3M makes no

representations as to its completeness or accuracy.  In addition,

information obtained from a database may not be as current as the

information in the MSDS available directly from 3M.