| fa |  |  |
| --- | --- | --- |
|  | Operational Safety Procedure Form (See ES&H Manual Chapter 3310 Appendix T1 Operational Safety Procedure (OSP) and Temporary OSP Procedure for instructions.) |  |

| Title: | Safe Operation of the Super Big Bite Power Supply |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: |  | Hall A |  |  |  |  | Type: |  OSP TOSP |  |
| Risk Classification (per Task Hazard Analysis attached) (See ESH&Q Manual Chapter 3210 Appendix T3 Risk Code Assignment.) |  |  |  | Highest Risk Code Before Mitigation |  |  |  | 3 |  |
|  |  |  |  | Highest Risk Code after Mitigation (N, 1, or 2): |  |  |  | N |  |
| Owning Organization: |  |  | Physics |  | Date: | 01/14/2019 |  |  |  |
| Document Owner(s): |  |  | Jack Segal |  |  |  |  |  |  |

DEFINE THE SCOPE OF WORK

- Purpose of the Procedure – Describe in detail the reason for the procedure (what is being done and why).

The intent of this procedure is to provide instruction for authorized personnel to safely operate, service, and troubleshoot the Super Big Bite Power supply.

- Scope – include all operations, people, and/or areas that the procedure will affect.

The use and safe operation of the Super Big Bite power supply to provide power for the Septum magnets.

- Description of the Facility – include building, floor plans and layout of the experiment or operation.

Water cooled 2200 amp, 290 volt DC power supply. Will be remotely controlled using existing software.

| ANALYZE THE HAZARDS and IMPLEMENT CONTROLS |  |
| --- | --- |
| - Hazards identified on written Task Hazard Analysis |  |
| See accompanying THA |  |
| - Authority and Responsibility: |  |
|  | - Who has authority to implement/terminate |
|  | Jack Segal Ethan Becker Joe Beaufait Heidi Fansler Jessie Butler |
|  | - Who is responsible for key tasks |
|  | Jack Segal Ethan Becker Joe Beaufait |
|  | - Who analyzes the special or unusual hazards including elevated work, chemicals, gases, fire or sparks (See ES&H Manual Chapter 3210 Appendix T1 Work Planning, Control, and Authorization Procedure) |
|  | Jack Segal Jessie Butler |
| - Personal and Environmental Hazard Controls Including: |  |
|  | - Shielding |
|  | None |
|  | - Barriers (magnetic, hearing, elevated or crane work, etc.) |
|  | None |
|  | - Interlocks |
|  | The Super Big Bite power supply faults will be interlocked back to EESSAF to prevent beam steering problems. |
|  | - Monitoring systems |
|  | Once the Super Big Bite power supply has been enabled the power supply can be controlled and monitored locally; or remotely with the MEDM GUI. |
|  | - Ventilation |
|  | - LCW (Low Conductivity Water) - Free Air (fans integral to the power supplies) |
|  | - Other (Electrical, ODH, Trip, Ladder) (Attach related Temporary Work Permits or Safety Reviews as appropriate.) |
| - List of Safety Equipment: |  |
|  | - List of Safety Equipment: |
|  | - Locks and Tags - Safety Glasses - VOM Meter - Necessary Arc Flash attire |
|  | - Special Tools: |
| - Associated Administrative Controls |  |
|  | Log all work done in the Hall A online log, the halog webpage |
| - Training |  |
|  | - What are the Training Requirements (See List of Training Skills) |
|  | To work on or about the power supply, one needs to have the necessary training to be an Electrical Worker - Lock Tag and Try (SAF104). - Electrical Safety (SAF603A, SAF603N1, SAF603N2). - Fire Prevention (SAF108). |

| DEVELOP THE PROCEDURE |  |
| --- | --- |
| - Operating Guidelines |  |
| Power supply should be operated within manufacturer’s guidelines and agreed upon limits set for experiment. |  |
| - Notification of Affected Personnel (who, how, and when include building manager, safety warden, and area coordinator) |  |
| Use the Hall A Tech on Call procedure. |  |
| - List the Steps Required to Execute the Procedure: from start to finish. |  |
| See accompanying THA |  |
| - Back Out Procedure(s) i.e. steps necessary to restore the equipment/area to a safe level. |  |
| Power down and turn off the power supply. Lock and tag out main power if authorized to do so. |  |
| - Special environmental control requirements: |  |
|  | - List materials, chemicals, gasses that could impact the environment (ensure these are considered when choosing Subject Mater Experts) and explore EMP-04 Project/Activity/Experiment Environmental Review below |
|  | None |
|  | - Environmental impacts (See EMP-04 Project/Activity/Experiment Environmental Review) |
|  | None |
|  | - Abatement steps (secondary containment or special packaging requirements) |
|  | None |
| - Unusual/Emergency Procedures (e.g., loss of power, spills, fire, etc.) |  |
| None |  |
| - Instrument Calibration Requirements (e.g., safety system/device recertification, RF probe calibration) |  |
| None |  |
| - Inspection Schedules |  |
| None |  |
| - References/Associated/Relevant Documentation |  |
| Equipment Manuals: https://hallaweb.jlab.org/tech/Detectors/public_html/magnets/super_big_bite/ |  |
| - List of Records Generated (Include Location / Review and Approved procedure) |  |
| None |  |

Submit Procedure for Review and Approval (See ES&H Manual Chapter 3310 Appendix T1 OSP & TOSP Instructions – Section 4.2 Submit Draft Procedure for Initial Review):

- Convert this document to .pdf

- Open electronic cover sheet:

https://mis.jlab.org/mis/apps/mis_forms/operational_safety_procedure_form.cfm

- Complete the form

- Upload the pdf document and associated Task Hazard Analysis (also in .pdf format)

Distribution: Copies to Affected Area, Authors, Division Safety Officer

Expiration:  Forward to ESH&Q Document Control

Form Revision Summary Revision 1.5 – 04/11/18 – Training section moved from section 5 Authority and Responsibility to section 9 Training Revision 1.4 – 06/20/16 – Repositioned “Scope of Work” to clarify processes Qualifying Periodic Review – 02/19/14 – No substantive changes required Revision 1.3 – 11/27/13 – Added “Owning Organization” to more accurately reflect laboratory operations. Revision 1.2 – 09/15/12 – Update form to conform to electronic review. Revision 1.1 – 04/03/12 – Risk Code 0 switched to N to be consistent with 3210 T3 Risk Code Assignment. Revision 1.0 – 12/01/11 – Added reasoning for OSP to aid in appropriate review determination. Revision 0.0 – 10/05/09 – Updated to reflect current laboratory operations ISSUING AUTHORITY; FORM TECHNICAL POINT-OF-CONTACT; APPROVAL DATE; REVIEW DATE; REV.. ESH&Q Division; Harry Fanning; 04/11/18; 04/11/21; 1.5. This document is controlled as an on line file. It may be printed but the print copy is not a controlled document. It is the user’s responsibility to ensure that the document is the same revision as the current on line file. This copy was printed on 7/30/2026.