|  | Task Hazard Analysis (THA) Worksheet (See ES&H Manual Chapter 3210 Appendix T1 Work Planning, Control, and Authorization Procedure) | Click For Word Doc |
| --- | --- | --- |

| Author: | Jack Segal |  |  |  |  | Date: |  | 01/14/2019 |  |  | Task #: If applicable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Complete all information. Use as many sheets as necessary |  |  |  |  |  |  |  |  |  |  |  |  |
| Task Title: |  | Safe Operation of the Super Big Bite Power Supply |  |  |  |  |  |  | Task Location: | Hall A |  |  |
| Division: |  | Physics |  |  | Department: |  | Hall A |  |  | Frequency of use: |  | For Apex experiment |
| Lead Worker: |  |  | Jack Segal |  |  |  |  |  |  |  |  |  |
| Mitigation already in place: Standard Protecting Measures Work Control Documents |  |  |  | None |  |  |  |  |  |  |  |  |

| Sequence of Task Steps | Task Steps/Potential Hazards | Consequence Level | Probability Level | Risk Code (before mitigation) | Proposed Mitigation (Required for Risk Code >2) | Safety Procedures/ Practices/Controls/Training | Risk Code (after mitigation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Electrical | Med | Low | 3 | - OSP. - Proper training & execution of approved procedures - Guard all exposed electrical connections | - Lock Tag and Try ( SAF104) - Electrical Safety (SAF603A, SAF603N1, SAF603N2) - Fire Protection (SAF108) | 0 |
|  | Magnetic Field | Med | Low | 1 | - OSP | - Measure and post 5 Gauss boundary - Warning Beacons | 0 |
|  | Fire | Low | Low | 1 | - OSP | - Hall A Technical Work Permit - Protection Systems | 0 |

| Highest Risk Code before Mitigation: | 3 | Highest Risk Code after Mitigation: | 0 |
| --- | --- | --- | --- |

When completed, if the analysis indicates that the Risk Code before mitigation for any steps is “medium” or higher (RC≥3), then a formal Work Control Document (WCD) is developed for the task.  Attach this completed Task Hazard Analysis Worksheet.  Have the package reviewed and approved prior to beginning work.  (See ES&H Manual Chapter 3310 Operational Safety Procedure Program.)

1. Define the scope of work

- Job Summary

This procedure provides guidelines on how to manage the commissioning, operation, and troubleshooting of the Septum magnet.

- Affected systems

Beam

Super Big Bite power supplies

Septum magnet

LCW

2. Analyze Hazard

This document

3. Develop and implement Hazard controls

Standard PPE

4. Identify danger zones

- Target and magnet access platform area

5. Training required

- Lock Tag and Try (SAF104)

- Fire Safety (SAF108)

- Electrical Safety (SAF603A, SAF603N1, SAF603N2)

6. Perform work within controls

- Number of people needed

1 or 2

- Verify that needed training is complete and current

- List of materials

VOM meter

Locks and Tags

DC Current meter

Temperature probe

- Procedure

- Perform pre-job briefing

- Ensure that LCW is on, that there is correct pressure (120 psi) and that there are no leaks

- Verify that all electrical connections are torqued and properly connected

- Check continuity and resistance of coil

- Check that all electrical connections are shielded

- Ensure that water and temperature are functioning correctly

- Remove from the area all ferromagnetic objects that could be affected

- Energize power supply and clear any faults

- Incrementally ramp current to power the magnet and check for hot spots at connections and on coils with temperature probe

- Also, verify that the current output from the Super Big Bite power supplies are stable and uniform between power supplies at the momentum settings for the power supplies

- In addition to current checks, verify with a Hall probe that the magnet’s B fields are the correct orientation and that the desired field uniformity and magnitude can be achieved

- Post job briefing and cleanup

Follow procedure

Document any mistakes, corrections, changes and pictures and implement changes

Turn in feedback to Hall work coordinator

7. Provide feedback and continuous improvement.

Form Revision Summary Periodic Review – 08/29/18 – No changes per TPOC Periodic Review – 08/13/15 – No changes per TPOC Revision 0.1 – 06/19/12 - Triennial Review. Update to format. Revision 0.0 – 10/05/09 – Written to document current laboratory operational procedure. ISSUING AUTHORITY; TECHNICAL POINT-OF-CONTACT; APPROVAL DATE; REVIEW DATE; REV.. ESH&Q Division; Harry Fanning; 08/29/18; 08/29/21; 0.1. This document is controlled as an on line file. It may be printed but the print copy is not a controlled document. It is the user’s responsibility to ensure that the document is the same revision as the current on line file. This copy was printed on 7/30/2026.