|  | Operational Safety Procedure Form (See ES&H Manual Chapter 3310 Appendix T1 Operational Safety Procedure (OSP) and Temporary OSP Procedure for instructions.) |
| --- | --- |

| Serial Number: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (Assigned by ESH&Q Document Control x7277) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | OSP X |  |  |  |  | TOSP |  |  |  |  |  |  |  | This document is written to mitigate hazard issues that are (check all that apply): Unable to comply with ES&H Manual requirements as written. New/anticipated/previously unrecognized. Determined to have an unmitigated Risk Code of 3 or 4. X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Issue Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (No more than three years from Issue Date except TOSP which is three months from Issue Date) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Title: | Maintenance, testing and diagnosis of the HRS Dipole power supplies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Location: |  |  | Hall A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Risk classification (See ESH&Q Manual Chapter 3210 Appendix T3 Risk Code Assignment.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Without mitigation measures (3 or 4): |  |  |  |  |  |  |  | 4 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | With mitigation measures in place (0, 1, or 2): |  |  |  |  |  |  |  | N |  |  |  |
|  | Document Owner(s): |  |  |  |  |  |  | Ed Folts, Jack Segal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 04-05-12 |  |  |  |  |  |  |  |
| Supplemental Technical Validations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard Reviewed (per ES&H Manual 2410-T1): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Subject Matter Experts Signature: |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |
|  | Bill Merz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Charles Hightower |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Approval Signatures: |  |  |  |  |  |  |  |  |  |  |  |  | Print Signature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |
|  | Division Safety Officer: |  |  |  |  |  |  |  | Javier Gomez |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Department or Group Head: |  |  |  |  |  |  |  |  |  |  | Robert Michaels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Safety Warden of Area: |  |  |  |  |  |  |  |  | Ed Folts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other Approval(s): |  |  |  |  |  | Bert Manzlak |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Document History: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Revision: |  | Reason for revision or update: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Serial number of superseded document |  |  |  |  |  |  |  |  |

Distribution: Copies to: affected area, authors, Division Safety Officer, ESH&Q Document Control

After expiration: Forward original and log sheet of trained personnel to ESH&Q Document Control.

|  | - Purpose of the Procedure |  |  |
| --- | --- | --- | --- |
|  | General safety guidance for the maintenance, testing and problem diagnosis of the Hall A HRS Dipole power supplies. |  |  |
|  | - Scope – include operations, people, and/or areas where procedure applies |  |  |
|  | Work on the Dipole power supply, its controls and interlocks. This OSP does not cover diagnostics which require operation of an energized supply with covers removed or doors, other than the door to the controls panel, open. Access to the power supply balcony. |  |  |
|  | - Description of the Facility: (include floor plans and layout of a typical experiment or operation) |  |  |
|  | Balcony at the rear of the HRS. |  |  |
|  | - Authority and Responsibility: |  |  |
|  |  | 4.1 Who has authority to implement/terminate |  |
|  | Ed Folts, Jack Segal |  |  |
|  |  | 4.2 Who is responsible for key tasks |  |
|  | Ed Folts, Jack Segal |  |  |
|  | - Who analyzes the special or unusual hazards (See ES&H Manual Chapter 3210 Appendix T1 Work Planning, Control, and Authorization Procedure) |  |  |
|  | Jack Segal |  |  |
|  | - Personal and environmental hazard controls including: |  |  |
|  |  | 6.1 Shielding |  |
|  | none |  |  |
|  |  | 6.2 Interlocks |  |
|  | none |  |  |
|  |  | - Other |  |
|  | Lock and Tag for electrical. In accordance with the standing Radiation Work Permit (RWP), Radcon personnel will survey the dust present for radioactive contamination and if required write a specific Radiation Work Permit for the job. |  |  |
|  | - Monitoring systems |  |  |
|  | none |  |  |
|  | - Ventilation |  |  |
|  | Standard hall ventilation |  |  |
|  | - List of safety equipment (i.e: personal protective equipment or special tools) |  |  |
|  | List of PPE for working in controls enclosure with 120V guards in place: none List of PPE for working in controls enclosure with 120V guards in removed: Safety glasses; Flame resistant long pants and long sleeve shirt List of PPE for safeing power supplies and performing diagnostics on the power supply: Safety glasses; Flame resistant long pants and long sleeve shirt; Meters rated to at least CAT IV for 480VAC. |  |  |
|  | - Associated administrative procedures |  |  |
|  | [Start Typing Here] |  |  |
|  | - Operating guidelines |  |  |
|  | [Start Typing Here] |  |  |
|  | - Notification of Affected Personnel (How and Who) |  |  |
|  | Affected personnel are on the power supply balcony or performing work near the magnet. Personnel executing the work on the power supply will notify affected personnel. Non-qualified, non-authorized personnel will vacate the area to a safe distance. The gate at the top of the access stairs to the balcony will be closed and the areas will be restricted to trained and necessary personnel. |  |  |
|  | - List of steps required to execute the procedure from start to finish. |  |  |
|  | De-powering and safeing the power supply ---------------------------------------------------------------------------------------------- - Perform pre-job briefing and ensure training of personnel. - Ramp the magnet power to zero amps and turn off the power supply. - Turn off the disconnect switch on the front of the power supply. - Using the VVU at the wall mounted disconnect switch, insure that there is power to the power supply. In the event the VVU is non-functional or gives unexpected results, back-out of the procedure and develop a plan to address the VVU issue. - Turn off power at the wall mounted disconnect switch and lock out the switch using lock and tag procedures. - Use the VVU to insure that the power has been shut down. - Unplug the power to the controls interface and lock it out using lock and tag procedures. The plug is marked with an appropriate tag. - Close the gate at the access to the power supply balcony. - Open the rear panel of the power supply and use the shorting probe to ground the magnet power cables and the capacitor banks at the capacitor grounding panel. - Install the grounding bar at the capacitor grounding panel. - Notify Radcon that the power supply is open and have it cleared by them prior to beginning work. - Post-job briefing and cleanup. Restarting the power supply ----------------------------------------------------------------------------------------- - Ensure that all components are reinstalled and secure. If the work was extensive, this should also be repeated by a second person. - Remove and secure the grounding bar. - Close and secure all doors. - Remove locks and turn on power at the wall mounted disconnect switch. - Unlock the controls plug and plug it in. - Reset the interlocks in the front rack. - Clear the interlocks on the front of the control panel. - Turn on the disconnect switch on the front of the power supply. - Clear the final interlocks and turn the power supply on. - Open the gate at the access to the balcony. - Ramp the power supply to the desired current. - Post-job briefing and cleanup. Working on controls (low voltage) ---------------------------------------------------------------------------------------------- - Perform pre-job briefing and ensure training of personnel. - Do not remove finger guards while the power supply is energized. - Post-job briefing and cleanup. Performing diagnostics ---------------------------------------------------------------------------------------------- - Perform pre-job briefing and ensure training of personnel. - Diagnostics includes measurements only. No manipulation of components is allowed during a diagnosis. - Post-job briefing and cleanup. |  |  |
|  | - Back out procedures, i.e., steps necessary to restore the equipment/area to a safe level. |  |  |
|  | As required for the task. |  |  |
|  | - Special environmental control requirements: |  |  |
|  | none |  |  |
|  | - Environmental Impacts (See EMP-04 Project/Activity/Experiment Environmental Review |  |  |
|  | none |  |  |
|  | - Abatement Steps – Secondary Containment, or Special Packaging requirements |  |  |
|  | none |  |  |
|  | - Training requirements |  |  |
|  | SAF603A and SAF 603N the basic electrical courses; SAF 802 Rad Worker II; SAF 104 Lock, Tag and Try; SAF105 CPR/AED USE; Equipment specific training |  |  |
|  | - Unusual/Emergency procedures e.g., Injury, Fire, Loss of power |  |  |
|  | Standard JLAB response |  |  |
|  | - Instrument calibration requirements, e.g., safety system/device recertification, RF probe calibration |  |  |
|  | none |  |  |
|  | - Inspection schedules |  |  |
|  | [Start Typing Here] |  |  |
|  | - References/Associated Documentation |  |  |
|  | [Start Typing Here] |  |  |
|  | - List of Records Generated (Include Location / Review and Approved procedure) |  |  |
|  | [Start Typing Here] |  |  |

Authorized/Trained Individuals:

| Print Name/Signature | Date |
| --- | --- |
| Ed Folts (“Gatekeeper”) |  |
| Jack Segal (“Gatekeeper”) |  |
| Heidi Fansler |  |
| Jessie Butler |  |
| Bill Merz |  |

Form Revision Summary Revision 1 – 12/01/11 - Added reasoning for OSP to aid in appropriate review determination. Revision 0 - 10/05/09 – Updated to reflect current laboratory operations ISSUING AUTHORITY; FORM TECHNICAL POINT-OF-CONTACT; APPROVAL DATE; EXPIRATION DATE; REV.. ESH&Q Division; Harry Fanning; 12/01/11; 12/01/14; 1. This document is controlled as an on line file. It may be printed but the print copy is not a controlled document. It is the user’s responsibility to ensure that the document is the same revision as the current on line file. This copy was printed on 7/30/2026.