1. Define the scope of work

Change out Moeller cooling hoses due to age and wear.

2. Analyze Hazard

| Task | Potential Hazards | Risk Code before mitigation | Safe Procedures/Practices/Controls | Risk Code after mitigation |
| --- | --- | --- | --- | --- |
| Moeller cooling hose change out. | Shock Radiation- contamination/ activation fall hazard cut/abrasion | 4 3 2 2 | LOTO, electrical work training, proper PPE. RCG coverage, Rad 2 training, PPE. Ladder training, harness training, PPE. Hand tool training, PPE | 1 1 0 0 |

3. Develop and implement Hazard controls

Follow procedure and task hazard analysis.

4. Perform work within controls (follow procedure)

- Summary of job

Moeller cooling hose change out.

- Affected systems

Moeller

- List of PPE

Rubber gloves, leather gloves, safety glasses, long sleeve cotton shirt,

- Identify danger zones

After radcon support

After LOTO

After fall protection mitigation

No danger zones noted

- Number of people needed

1 person to change hoses

Radcon support

- List of materials

Rubber hose, hose clamps

- Procedure

Pre job briefing

LOTO electrical system

Tag out water system (valves closed)

Remove electrical safety guards

Cut hoses with razor knife and remove hose

Measure length of original hose and cut new hose to length

Heat hose ends with heat gun

Install hose clamps on hose ends

Push hose ends on hose barbs

Tighten hose clamps

- Post job briefing and cleanup

Pressurize cooling system and check for leaks

If  leaks exist tighten hose clamp or replace O ring

Re install magnet guards

Remove LOTO plug in power cords using proper PPE

5. Provide feedback and continuous improvement. (Document any mistakes, corrections, changes and pictures and implement changes.)