1. Define the scope of work

Remove the SC2 wire chamber from the Left Detector Stack.

2. Analyze Hazard

| Task | Potential Hazards | Risk Code before mitigation | Safe Procedures/Practices/Controls | Risk Code after mitigation |
| --- | --- | --- | --- | --- |
| Remove the SC2 wire chamber from the Left Detector Stack. | Abrasions, contusions Damage to detectors Trip hazards Damage to cables | 1 3 2 1 | Attentive work practices Install appropriate covers, Attentive work practices. Attentive work practices Secure cables out of the way. | 0 2 1 0 |

3. Develop and implement Hazard controls

Follow procedure and task hazard analysis.

4. Perform work within controls (follow procedure)

- Summary of job

- Prep the SC2 wire chamber and Service Platform. Remove SC2 and place on storage stand.

- Affected systems

- Left and Right Spectrometers’ motion control, overhead crane, PSS, high voltage for wire chamber, FPP gas system, left side of Left Arm

Shield house doors must be open far enough that there is interference if both spectrometers are at small angles

Detector Stack on the Service Platform adds an interference for crane use

PSS system must have the Shield House doors interlock jumpered when there is a test of the PSS.  Shield House doors cannot be closed with the Detector Stack out on the Service Platform.

High voltage power supply channels for the wire chamber must be turned off before the high voltage cables are disconnected

SC2 gas supply should be valved off, at the panel at the front of the Detector Stack, before the gas lines are disconnected.

Work that risks dropping something from the Service Platform should not be done while personnel are in the area on the floor

- List of PPE

- Hard hat, safety shoes

- Fall protection harness if arial lift used

- Identify danger zones

- Detector Service Platform

- Left side of Left Spectrometer

- Number of people needed

- 2 at minimum, beyond that is at the discretion of the person in charge of the lift

- List of materials

- One 4 foot, two 8 foot, and one 10 foot slings

- Two 1/2 inch shackles

- 1/2 ton chainfall

- Laser plumb bob, if desired

- Red support beam with modified jackstand

- Appropriate rail extensions

- Sheet aluminum covers for SC2

- Procedure

- Notify Hall A Work Coordinator when the task will be undertaken

- Pre job briefing

- Position red support beam over the bayonet box

- Remove grounding cable to the detector, used to ground the detector to the Detector Stack

- Remove C-clamps on the bottom rail, one on each side of the bottom roller assembly to block the detector from rolling to either side

- Ensure the 1/4-20 screw (it is a hard stop) is removed from the left end of the bottom rail

- Insert appropriate rail extension into the left end of the bottom rail

- Place modified jackstand support on the red beam, under the rail extension. Adjust to support the rail extension in a horizontal position.

- Insert appropriate rail extension into the left end of the top rail. We have, at present, two methods of doing this:

Use a 10 foot sling doubled over in a choke at the C.G. of the rail extension. This gives enough play in the sling that a person on the top electronics platform can guide the rail extension into the top rail. Remove the sling after the rail extension is fully inserted into the top rail.

Bring the arial lift alongside the spectrometer. Take the rail extension up in the basket and manually insert it into the top rail.

- Roll detector out onto the rail extensions. Pay attention to the roller placement on the top rail extension. If the rail extensions spread apart it is possible for the top roller to slip off the top rail extension.

- It has proven very unwise to attempt to install or remove the cover for SC2 when it is up by the Detector Stack. Although it is very dangerous for the straw tubes, I recommend removing/installing the cover when it is down by the floor. Use a LOT of EXTRA care when rigging SC2, moving it, and removing the rigging without the cover in place.

- Attach rigging to the detector. Place laser plumb-bob on crane block if desired. Measured weight of SC2 (no power cables attached) is 763 lbs.. One 10 foot sling in an inverted basket on the top of chamber. 1/2" shackles around bolts on top rollers. 4' sling in a basket to 1/2 ton chainfall. Two 8' slings in a choke on the bottom come up to the chainfall. The choke is done so the bite is at the bottom of the detector frame. This keeps the chain of the chainfall as far away from the tubes as I can get them. The wire chambers are not built symmetrically. The left side, as the beam sees it, is a little heavier than the right side. Offset the slings on the bottom of the chamber to compensate for this. Something like 4 to 6 inches of offset is sufficient.

- This setup just barely keeps the chain of the chainfall from hitting the strawtubes. It may be prudent to tie the chains to the 8 foot slings to prevent them from damaging the tubes.

  - Lift the detector off the rail extensions

  - Set the detector down on the support stand

  - Remove the rigging from the detector

- Post job briefing and cleanup

  - Put all tools away

  - Remove rigging, rail extensions and modified jackstand

  - Notify Hall A Work Coordinator when the task is completed

5. Provide feedback and continuous improvement. (Document any mistakes, corrections, changes and pictures and implement changes.)