ACP & D Limited

APS MANUAL

_________________________________________________________

Handbuch

STEPPING MOTOR DRIVER

SERIES

STAR 2000

Mod.

APS x-B-00

ver. 01/02

RELEASE: 50FW1H  ;  60FW2B

###### Date: 13.02.2003

INDEX:

- TECHNICAL DATA……………………………………………………………………………….Pag. 5

  - Power supply / output current

  - Serial interface

  - Inputs and outputs

  - Protections

  - Mechanical dimensions

- DRIVE CONNECTION…………...……………………………………………………………….Pag. 8

  - Drive layout

  - Installation notes

  - Power supply

  - Motor connection

  - Inputs/outputs connections

  - Inputs/outputs diagram and connection notes for standard 99 version

  - Inputs/outputs diagram and connection notes for standard 2000 version

  - RS232 serial interface

  - RS232 connections diagram

  - RS422 serial interface (RS485 full duplex)

  - RS485 serial interface (half duplex)

  - RS485/RS422 connection diagram

- JUMPER SETTINGS……………….……………………………………………………………….Pag. 16

- OPERATING MODE…………………...…………………………………..……………………….Pag. 16

- STANDARD MODE……………….……………………………………………………….……….Pag. 17

  - Current regulation

  - DIP-SWITCHES settings

- SERIAL MODE………...…………..………………...……………………………………..……….Pag. 20

  - Current regulation

  - Serial interface monitor

  - Switches and communication interface settings

  - Transmission timing of serial commands

  - Communication protocol

6.5.1 Single address command

6.5.2 Multi address command

6.5.3 Command addressed to all drives

6.5.4 Commands protocol

6.5.5 Examples of commands / example of calculation ByteChecksum

6.5.6 Simple motion program

1. TECHNICAL DATA

1.1 POWER SUPPLY/OUTPUT CURRENT

| SIZE | APS1 | APS2 | APS3 | APS4 | APS5 |
| --- | --- | --- | --- | --- | --- |
| Vdc nom. [V] | From 40 to 80 | From 40 to 80 | From 40 to 80 | From 80 to 140 | From 160 to 180 |
| Vdc max. [V] | 90 | 90 | 90 | 155 | 195 |
| Vdc min. [V] | 30 | 30 | 30 | 75 | 155 |
| I max. [A] | 4 | 6 | 10 | 12 | 10 |
| I min. [A] | 0.4 | 1 | 1 | 1 | 1 |
| I step [A] | 0.2 | 0.5 | 0.5 | 0.5 | 0.5 |
| Operating temperature [°C] | 0-55 | 0-55 | 0-55 | 0-55 | 0-55 |

PARAMETERS DESCRIPTION

VDC nom.:	Rated value of voltage by which the drive can be powered.

VDC max.:	Maximum voltage at which the drive can operate. Over this limit, the protection of maximum voltage inhibits the drive.

VDC min.:	Minimum voltage at which the drive can operate. Under this limit, the protection of minimum voltage inhibits the drive.

I max.:	Maximum value of  phase current.

I min.:	Minimum value of  phase current.

I step:	Spacing of the current values.

Operation

temperature:	For any current over 6 Amps, a forced ventilation is necessary.

1.2 SERIAL INTERFACE

Drives are supplied with RS 232 and RS 485 serial interface selecting by DIP-SWITCH.

The communication protocol is on board and in the following described.

With RS232 serial interface is possible to connect only one drive.

With the RS485/RS422 serial interface is possible to connect up to 32 drives, identificated by address set through dip-switches.

1.3 INPUTS  AND  OUTPUTS

Inputs are PNP, outputs are PNP open collector optoisolated (10mA max) on STANDARD 99 version.

Inputs are PNP or NPN, outputs are PNP open collector optoisolated (10mA max) on STANDARD 2000 version.

INPUTS FEATURES:

| INPUT VOLTAGE | IN1 HIGH SPEED INPUT | IN2, IN3, ENABLE INPUTS |
| --- | --- | --- |
| LOW LEVEL | From 0 V to 8 V | From 0 V to 2.5 V |
| HIGH LEVEL | From 11 V to 30 V | From 4.6 V to 30 V |

INPUT SIGNALS:

HIGH SPEED INPUT	: Max. frequency 50 KHz

Min. impulse width  t=10 sec

INPUTS			: Max. frequency 5KHz

Min. impulse width t=100 sec

OUTPUTS FEATURES:

| OUTPUT VOLTAGE | Load 5mA | Load 10mA |
| --- | --- | --- |
| OUT ON COM.OPTO-OUT=12V COM.OPTO-OUT=24V | 11.5 V 23.5 V | 11.2 V 23.2 V |
| OUT OFF | 0 V | 0 V |

1.4 PROTECTIONS

Drive is provided with protections against overtemperature, overvoltage, undervoltage, short-circuits among  outputs and also among outputs and the positive power pole. If one of the mentioned conditions occurs, drive disables the power bridge and shows an error condition on the display.

‘u’	- Power supply volts out of correct limits

‘t’	- Thermic protection event occurred

‘c’	- Overcurrent protection event occurred

If  drive is ready,  display shows the letter ‘r’ (ready).

1.5 MECHANICAL DIMENSIONS

APS 1-B-00 / APS 2-B-00 / APS 3-B-00:

vent (optional)

APS 4-B-00 / APS 5-B-00:

2. DRIVE CONNECTION

2.1 DRIVE LAYOUT

2.2 INSTALLATION NOTES:

WARNING

DANGER OF ELECTRICAL SHOCK . ONLY QUALIFIED INDIVIDUALS SHOULD WORK ON THIS EQUIPMENT. DISCONNECT ALL POWER BEFORE WORKING ON EQUIPMENT. DANGEROUS VOLTAGES MAY EXIST AFTER POWER IS REMOVED! CHECK DC BUS VOLTAGE OF DRIVES EACH TIME POWER IS REMOVED BEFORE WORKING ON EQUIPMENT.

2.3 POWER SUPPLY:

| CONNECTION | SIGNAL | FUNCTION |
| --- | --- | --- |
| J1 - Pin 1 | +HV | +VDC power supply (see technical data) |
| J1 - Pin 2 | GND | 0 V power supply |

The drive needs an external power supply that must be bilt as indicated in the FIG.1 otherwise use the TPS power supply (see TPS handbook).

FIG.1

To reduce EMI/RFI it is advisable:

Maximum wire reduction between power supply and drive, between drive and motor and, eventually, using shielded wires and insert the capacitor C1 (100nF 200 V, adeguate for switching applicatoins).

The C2 capacitor value is 470F 100V for APS1,APS2 and APS3, 160V for APS4 and 200V for APS5 for every Amps requied.

i.e.:  for 1 APS3/A (10A) use 4700F 100V. For 1 APS4 (12A) use 4700 F 160V. For 1 APS5 (10A) use 4700 F 200V.

The transformer  power is P=Vac*(Inf(tot) + 1)

Where P is VA power, Vac is secondary voltage in Volts and Inf(tot) is the sum of all nominal currents set in all the drive to be supplied.

2.4 MOTOR CONNECTION:

| CONNECTION | SIGNAL | FUNCTION |
| --- | --- | --- |
| J1 - Pin 3 | PHASE A1 | Phase A1 Step motor |
| J1 - Pin 4 | PHASE A2 | Phase A2 Step motor |
| J1 - Pin 5 | PHASE B1 | Phase B1 Step motor |
| J1 - Pin 6 | PHASE B2 | Phase B2 Step motor |

2.5 INPUTS/OUTPUTS CONNECTIONS:

Regarding the use of the drive the pins of the connections have different meanings:

| CONNECTION | SIGNAL | STANDARD MODE FUNCTION | SERIAL MODE FUNCTION |
| --- | --- | --- | --- |
| J2C - Pin 21 | +15V | +15V output | +15V output |
| J2C - Pin 20 | GNDL | Logic signals GND | Logic signals GND |
| J2C - Pin 19 | Unassigned |  |  |
| J2C - Pin 18 | COM.IN | Common inputs optocouplers | Common inputs optocouplers |
| J2C - Pin 17 | DISABLE | DISABLE input | DISABLE input |
| J2C - Pin 16 | IN3 | CURRENT REDUCTION input | IN3, programmabile input |
| J2C - Pin 15 | IN2 | DIRECTION input | IN2, programmabile input |
| J2B - Pin 14 | IN1 | STEP-IN input | IN1, programmabile input |
| J2B - Pin 13 | Unassigned |  |  |
| J2B - Pin 12 | COM.OUT | Common outputs optocouplers | Common outputs optocouplers |
| J2B - Pin 11 | OUT2 | DRIVER READY output | DRIVE-READY output |
| J2B - Pin 10 | OUT1 | Unassigned | IN-POSITION output |
| J2B - Pin 9 | RIF. +10V | Unassigned | Unassigned |
| J2B - Pin 8 | ANALOG.IN | Unassigned | Unassigned |
| J2A - Pin 7 | GNDL | Logic signals GND | Logic signals GND |
| J2A - Pin 6 | GNDL | Logic signals GND | Logic signals GND |
| J2A - Pin 5 | +5V | Unassigned | Unassigned |
| J2A - Pin 4 | Unassigned |  |  |
| J2A - Pin 3 | Unassigned |  |  |
| J2A - Pin 2 | Unassigned |  |  |
| J2A - Pin 1 | Unassigned |  |  |

2.6 INPUTS/OUTPUTS DIAGRAM FOR STANDARD 99 VERSION

###### FIG.2

INPUT/OUTPUTS CONNECTION NOTES (STANDARD 99 VERSION):

JP 1	- When inserted, it associates the input common pole with GND of the drive

(non-optoisolated PNP inputs)

JP 1	- When not inserted, it associates the input optoisolated. In this case, you must connect the GND of the external power supply to the connector J2, pin 18.

Outputs are always optoisolated. Therefore, an external powering (from 12VDC to 24VDC) must be connected to the common  pole  of the optoisolators (Pin 12, conncector J2)

2.7 INPUTS/OUTPUTS DIAGRAM FOR STANDARD 2000 VERSION

###### FIG.2

INPUT/OUTPUTS CONNECTION NOTES (STANDARD 2000 VERSION):

JP 1	- When inserted in 1-2 position, it associates the input common pole with GND of the drive

(non-optoisolated PNP inputs)

JP 1	- When inserted in 2-3 position, it associates the input common pole with +15V of the drive

(non-optoisolated NPN inputs)

JP 1	- When not inserted, it associates the input optoisolated. In this case, you must connect the GND of the external logic power supply to the connector J2 - pin 18 for PNP inputs, or the positive (+12VDC/+24VDC) of external logic power supply for NPN inputs.

Outputs are always optoisolated. Therefore, an external powering (from 12VDC to 24VDC) must be connected to the common  pole  of the optoisolators (Pin 12, conncector J2)

2.8 RS232 SERIAL INTERFACE:

J4 has two function as the follow way: One for RS232 connection and the other for firmware upgrade.

For firmware upgrade refer to “STAR 2000 UPGRADE HANDBOOK”.

| LEAD | SIGNAL | FUNCTION |
| --- | --- | --- |
| J4 - Pin 6 | TXD1 | RS232 TX signal |
| J4 - Pin 5 | RXD1 | RS232 RX signal |
| J4 - Pin 4 | /RES | RESET signal – ONLY FOR FIRMWARE UPGRADE |
| J4 - Pin 3 | GNDL | RS232 signals GND |
| J4 - Pin 2 | VPPIN | VPP input – ONLY FOR FIRMWARE UPGRADE |
| J4 - Pin 1 | +Vo | Vo output – ONLY FOR FIRMWARE UPGRADE |

2.9 RS232 CONNECTIONS DIAGRAM:

Connectione diagram between STAR 2000 and  RS232 standard connector.

###### FIG.3

2.10 RS422 SERIAL INTERFACE (RS485 full duplex):

For RS422 serial interface use J3 connector

| CONNECTION | SIGNAL | FUNCTION |
| --- | --- | --- |
| J3 - Pin 10 | GNDL OUT | RS422 signals GND. Output for other drives |
| J3 - Pin 9 | GNDL IN | RS422 signals GND |
| J3 - Pin 8 | +TX485 OUT | RS422 +TX signal. Output for other drives |
| J3 - Pin 7 | +TX485 IN | RS422 +TX signal |
| J3 - Pin 6 | - TX485 OUT | RS422 –TX signal. Output for other drives |
| J3 - Pin 5 | - TX485 IN | RS422 –TX signal |
| J3 - Pin 4 | +RX485 OUT | RS422 +RX signal. Output for other drives |
| J3 - Pin 3 | +RX485 IN | RS422 +RX signal |
| J3 - Pin 2 | - RX485 OUT | RS422 +RX signal. Output for other drives |
| J3 - Pin 1 | - RX485 IN | RS422 +RX signal |

2.11 RS485 SERIAL INTERFACE (half duplex):

For RS485 serial interface use J3 connector

| CONNECTION | SIGNAL | FUNCTION |
| --- | --- | --- |
| J3 - Pin 10 | GNDL OUT | RS485 signals GND. Output for other drives |
| J3 - Pin 9 | GNDL IN | RS485 signals GND |
| J3 - Pin 8 | +TX485 OUT | RS485 +TX/+RX signals. Output for other drives |
| J3 - Pin 7 | +TX485 IN | RS485 +TX/+RX signals |
| J3 - Pin 6 | - TX485 OUT | RS485 –TX/–RX signals. Output for other drives |
| J3 - Pin 5 | - TX485 IN | RS485 –TX/–RX signals |
| J3 - Pin 4 | +RX485 OUT | Not use |
| J3 - Pin 3 | +RX485 IN | Not use |
| J3 - Pin 2 | - RX485 OUT | Not use |
| J3 - Pin 1 | - RX485 IN | Not use |

2.12 RS485/RS422 SERIAL CONNECTION DIAGRAM:

Connection diagram on J3 connector

###### RS485 FULL DUPLEX CONNECTION

###### FIG.4

To use RS485 full duplex (RS422) set JP4 jumper between 1-2  position (factory default), see layout par.2.1 and FIG.4

###### RS485 HALF DUPLEX CONNECTION

###### FIG.5

To use RS485 half duplex  set JP4 jumper between 2-3 position, see layout par.2.1 and FIG.5

3. JUMPERS SETTINGS

STANDARD 99 VERSION:

JP 1	- When inserted, it associates the input common pole with GND of the drive (non-optoisolated inputs)

JP 1	- When not inserted, it associates the input optoisolated. In this case, you must connect the GND of the external power supply to the connector J2, pin 18.

JP 2	- unassigned

JP 4	- When inserted into 1-2 position  RS485 serial interface is on full duplex mode

- When inserted into 2-3 position RS 485 serial interface is on half duplex mode.

JP 5, JP 6	- When inserted, they add termination resistors (120 Ohm) needed for the last drive of the chain, between the  signals TX+,TX- and RX+,RX- of the RS485 serial interface.

Factory configuration: JP1 inserted;  JP2 not inserted;  JP4 inserted  in 1-2 position;  JP5, JP6 not inserted.

STANDARD 2000 VERSION:

JP 1	- When inserted in 1-2 position, it associates the input common pole with GND of the drive

(non-optoisolated PNP inputs)

JP 1	- When inserted in 2-3 position, it associates the input common pole with +15V of the drive

(non-optoisolated NPN inputs)

JP 1	- When not inserted, it associates the input optoisolated. In this case, you must connect the GND of the external logic power supply to the connector J2 - pin 18 for PNP inputs, or the positive (+12VDC/+24VDC) of external logic power supply for NPN inputs.

For JP2,JP4,JP5 and JP6 the functions is the same of the STANDARD 99 version.

Factory configuration: JP1 inserted in 1-2 position;  JP2 not inserted;  JP4 inserted  in 1-2 position;  JP5, JP6 not

inserted.

4. OPERATING MODE

Drive can be used in two operation modes:

A – STANDARD MODE		: The drive has used with step and direction

B – SERIALE MODE  		: The drive has used with RS232 or RS485/RS422 serial interface

To set this modes use DIP B-1

DIPB 1 – OFF:	 STANDARD mode

–  ON :	 SERIAL mode

When the mode is set to SERIAL, to change serial type use DIP B-3:

DIPB 3	– OFF :	 RS485/RS422 interface (J3 connector)

–  ON  :	 RS232             interface (J4 connector)

THIS SELECTIONS MUST BE DONE BEFORE POWERING THE DRIVE.

5. STANDARD MODE

In this mode the drive has used with step and direction. The inputs/outputs are the follows:

INPUTS:

| ###### SIGNAL | ###### FUNCTION |
| --- | --- |
| STEP-IN (J2-14) | Execute the step on the LOW/HIGH transition of this signal. Use a square wave with duty-cycle of 50%. Signal absence for 0.5 seconds determines the automatic current reduction (stand-by condition). Percentage of reduction in stand-by can be set to 25% or to 50% of the regulated current through Dip-switch A-1. |
| DIRECTION (J2-15) | Select the motor wise Signals must be stable for at least 50 microsecunds before and 50 microsecunds after the low/high transition of the STEP-IN signal. |
| CURRENT REDUCTION (J2-16) | It reduces the motor current. The percentage of current reduction can be set from 25% or 50% of the regulated current through Dip-switch A-1. |
| ###### DISABLE (J2-17) | It disables the power bridge. |

OUTPUTS:

| ###### SIGNAL | ###### FUNCTION |
| --- | --- |
| ###### OUT1 (J2-10) | Unassigned |
| OUT2 (J2-11) | DRIVER_READY. Open collector (10 mA max.) Drive in protection: OUT OFF (Low level) Drive ready : OUT ON (High level) |

5.1 CURRENT REGULATION:

For setting current proceed as follows:

- Set dip-switch B-4 to ON (current regulation mode).

- Turn RV6 trimmer  until  display shows the required current (CW to increase).

- Regulation field: from 1A to 10 A at steps of  0.5A for APS3/APS5 drive.

Regulation field: from 1A to 12 A at steps of  0.5A for APS4 drive.

Regulation field: from 1A to 6 A at steps of  0.5A for APS2 drive.

Regulation field: from 0.4A to 4 A at steps of  0.2A for APS1 drive.

- Set dip-switch B-4 to OFF  (Run mode).

Table for setting current values and relating values shown on the display of drive APS:

| DISPLAYED VALUE | ###### SETTING ###### CURRENT ###### APS 1 | ###### SETTING ###### CURRENT APS 2 | ###### SETTING ###### CURRENT APS4 | ###### SETTING ###### CURRENT APS 3 / APS5 |
| --- | --- | --- | --- | --- |
| 1 | 0.4 A | 1.0 A | 1.0 A | 1.0 A |
| 1. | 0.6 A | 1.5 A | 1.5 A | 1.5 A |
| 2 | 0.8 A | 2.0 A | 2.0 A | 2.0 A |
| 2. | 1.0 A | 2.5 A | 2.5 A | 2.5 A |
| 3 | 1.2 A | 3.0 A | 3.0 A | 3.0 A |
| 3. | 1.4 A | 3.5 A | 3.5 A | 3.5 A |
| 4 | 1.6 A | 4.0 A | 4.0 A | 4.0 A |
| 4. | 1.8 A | 4.5 A | 4.5 A | 4.5 A |
| 5 | 2.0 A | 5.0 A | 5.0 A | 5.0 A |
| 5. | 2.2 A | 5.5 A | 5.5 A | 5.5 A |
| 6 | 2.4 A | 6.0 A | 6.0 A | 6.0 A |
| 6. | 2.6 A | - | 6.5 A | 6.5 A |
| 7 | 2.8 A | - | 7.0A | 7.0A |
| 7. | 3.0 A | - | 7.5 A | 7.5 A |
| 8 | 3.2 A | - | 8.0 A | 8.0 A |
| 8. | 3.4 A | - | 8.5 A | 8.5 A |
| 9 | 3.6 A | - | 9.0 A | 9.0 A |
| 9. | 3.8 A | - | 9.5 A | 9.5 A |
| 0 | 4.0 A | - | 10.0 A | 10.0 A |
| 0. | - | - | 10.5 A | - |
| a | - | - | 11.0 A | - |
| a. | - | - | 11.5 A | - |
| b | - | - | 12.0 A | - |

NOTE: ADJUST CURRENT WHEN MOTOR IS HOLDING.

5.2 DIP-SWITCHES SETTINGS:

| #### DIP SWITCH A – STANDARD MODE |  |  |
| --- | --- | --- |
| ##### DIP | ###### ON | ###### OFF |
| 6 | 1/16 of step (3200 steps/rev) only for STANDARD 2000 version | Other resolution selectables with DIPB 2-3 Only for STANDARD 2000 version |
| 5 | Unassigned | Unassigned |
| 4 | Unassigned | Unassigned |
| 3 | Fast decay enabled - low noise mode only for STANDARD 2000 version | Fast decay disabled – normal mode only for STANDARD 2000 version |
| 2 | Current damp enabled (to skip motor resonance) only for STANDARD 2000 version | Current damp disabled only for STANDARD 2000 version |
| 1 | Current reduction in stand-by, set to 50% of the setting current | Current reduction in stand-by, set to 25% of the setting current |

| #### DIP SWITCH B – STANDARD MODE |  |  |
| --- | --- | --- |
| ##### DIP | ###### ON | ###### OFF |
| 4 | Current regulation mode | RUN mode |
| 1 | Operating in serial mode (to be set before powering the drive) | Operating in standard mode (to be set before powering the drive) |

| #### DIP SWITCH B – STANDARD MODE |  |  |
| --- | --- | --- |
| ##### DIPB 2 | ###### DIPB 3 | ###### STEP DIVISION |
| ON | OFF | Full step (200 steps/rev.) |
| ON | ON | Half step (400 steps/rev.) |
| OFF | ###### OFF | Quarter of step (800 steps/rev.) |
| OFF | ON | Eighth of step (1600 steps/rev.) |

6. SERIAL MODE

In this mode the drive has used with serial interface. The inputs/outputs are the follows:

INPUTS:

| ###### SIGNAL | ###### FUNCTION |
| --- | --- |
| IN1 (J2-14) | Programmable via serial command. |
| IN2 (J2-15) | Programmable via serial command. |
| IN3 (J2-16) | Programmable via serial command. |
| ###### DISABLE (J2-17) | It disables the power bridge |

OUTPUTS:

| ###### SIGNAL | ###### FUNCTION |
| --- | --- |
| ###### OUT 1 (J2-10) | IN-POSITION output: Motor is holding : OUT OFF (Low level) Motor running : OUT ON (High level) The levels shown are the default levels. They can be inverted through a serial command (see 0x2B) |
| OUT 2 (J2-11) | DRIVE-READY output: Drive in protection: OUT OFF (Low level) Drive ready : OUT ON (High level) |

6.1 CURRENT REGULATION:

Motor current can be set by on board available RV6 trimmer, as described  in standard mode: besides this value can be changed through a serial command.

RV6 trimmer setting value is acquired by the processor at ‘power on’ or at current regulation setting.

Current setting through serial command will remain available only until drive will be unpowered or until current regulation will be set through the trimmer.

Therefore trimmer can be used for setting a default current at ‘power on’, subsequently current can be set  to a different value through serial command.

6.2 SERIAL INTERFACE MONITOR:

On the display we can see the correct working of serial interface. The decimal point indicate when a command is received (RX), the nearest horizontal segment indicate when a answer is sended (TX).

6.3 SWITCHES AND COMMUNICATION INTERFACE SETTINGS

Communication parameters:

BAUD RATE	: 9600 (DIP A-1  ON) or 19200 (DIP A-1  OFF)

PARITY	: NO PARITY

DATA BITS	: 8

BIT STOP	: 1

DRIVE IDENTIFICATION ADDRESS SETTINGS ON ‘A’ DIP-SWITCH

| DIPA-2 (BIT4) | DIPA-3 (BIT3) | DIPA-4 (BIT2) | DIPA-5 (BIT1) | DIPA-6 (BIT0) | ADDRESS |
| --- | --- | --- | --- | --- | --- |
| OFF | OFF | OFF | OFF | OFF | 0 |
| OFF | OFF | OFF | OFF | ON | 1 |
| OFF | OFF | OFF | ON | OFF | 2 |
| OFF | OFF | OFF | ON | ON | 3 |
| OFF | OFF | ON | OFF | OFF | 4 |
| OFF | OFF | ON | OFF | ON | 5 |
| OFF | OFF | ON | ON | OFF | 6 |
| OFF | OFF | ON | ON | ON | 7 |
| OFF | ON | OFF | OFF | OFF | 8 |
| OFF | ON | OFF | OFF | ON | 9 |
| OFF | ON | OFF | ON | OFF | 10 |
| OFF | ON | OFF | ON | ON | 11 |
| OFF | ON | ON | OFF | OFF | 12 |
| OFF | ON | ON | OFF | ON | 13 |
| OFF | ON | ON | ON | OFF | 14 |
| OFF | ON | ON | ON | ON | 15 |
| ON | OFF | OFF | OFF | OFF | 16 |
| ON | OFF | OFF | OFF | ON | 17 |
| ON | OFF | OFF | ON | OFF | 18 |
| ON | OFF | OFF | ON | ON | 19 |
| ON | OFF | ON | OFF | OFF | 20 |
| ON | OFF | ON | OFF | ON | 21 |
| ON | OFF | ON | ON | OFF | 22 |
| ON | OFF | ON | ON | ON | 23 |
| ON | ON | OFF | OFF | OFF | 24 |
| ON | ON | OFF | OFF | ON | 25 |
| ON | ON | OFF | ON | OFF | 26 |
| ON | ON | OFF | ON | ON | 27 |
| ON | ON | ON | OFF | OFF | 28 |
| ON | ON | ON | OFF | ON | 29 |
| ON | ON | ON | ON | OFF | 30 |
| ON | ON | ON | ON | ON | 31 |

NOTE: If  several drives are connected on RS485 serial line, make sure  that all drives have a different address.

| #### DIP SWITCH B – SERIAL MODE |  |  |
| --- | --- | --- |
| ##### DIP | ###### ON | ###### OFF |
| 4 | Current regulation mode | RUN mode |
| 3 | RS-232 serial interface (set before drive powering) | RS-485 serial interface (set before drive powering) |
| 2 | Damping deactivated | Damping activated (for damping the mechanical motor resonances) |
| 1 | Serial mode (set before drive powering) | Standard mode (set before drive powering) |

6.4 TRANSMISSION TIMING OF SERIAL COMMANDS:

Following instructions must be followed by sending any serial command to the drive:

COMMANDS WITH ANSWER:

- Send command to drive

- Wait for answer from drive

- Send next command

- Wait for answer from drive

COMMANDS WITHOUT ANSWER:

- Send command to drive

- Delay of at least 5 ms (for command execution)

- Send next command

- Delay of at least 5 ms (for command execution)

Sometime the answer of the drive is too fast for the control (PC or PLC) in this case it’s necessary to setting an “answer delay” command. (for example 5ms)

6.5 COMMUNICATION  PROTOCOL

Systems can  use a single drive or  several drives, which are connected in multidrop to RS485 serial line (full duplex). Commands can be sent either to a definite drive (by specifying its address in the string command) or to all drives.

In the first case drive will answer to the command; in the second case  no reply will be given.

6.5.1 SINGLE ADDDRESS COMMAND:

DATA TO BE SENT TO THE DRIVE:

Commands succession to be sent to the drive must respect following structure:

| Byte_start | byte_nbyte_address |  |  |  |  |  |  |  | byte command | [byte_par0] | [byte_par1] | byte_checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BYTE [0xFC] | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | BYTE | BYTE | BYTE | BYTE |
|  | nbyte |  |  | drive_address |  |  |  |  |  |  |  |  |

byte_start		: 0xFC. This byte means that a command will be sent to one drive only,

byte_nbyte_address	: This byte contains two indications:

- drive_address	The first 5 bits (from bit0 to bit4) contain the drive address (from 0 to 31).

- nbyte 	The following 3 bits (from bit5 to bit7) contain the bytes numbers which

follow byte_nbyte_address before sending the byte_checksum.

byte_command		: This byte represents the command (see  commands protocol ).

byte_par0, byte_par1	: The bytes, which follow the byte_command, represent the sending command parameters.

byte_checksum                 : This byte must be calculated by the user as complement of the less significant byte

resulting from the sum of all sending bytes (including the byte_start), in order to have, as final result, one byte only. The function of this byte is to verify the correct transmission command (see example in paragraph  6.5.5).

DRIVE ANSWER:

Wrong or  not foreseen command transmission, drive will answer  byte-nak (0x15).

Correct command transmission, drive will answer byte_ack  (0x06) and it will be followed by an answer bytes series as forseen in the above mentioned  format.

EXAMPLES:

- To send RESET command to drive 0:

Command string: 0xFC 0x20 0x01 0xE2		drive answer: 0x06

- To send RESET command to drive 1:

Command string: 0xFC 0x21 0x01 0xE2		drive answer: 0x06

6.5.2 MULTI ADDRESSES COMMAND:

DATA TO BE SENT TO THE DRIVES:

Commands succession to be sent to the drive must respect  following structure:

| byte_start | byte_nbyte_address |  |  |  |  |  |  |  | byte_multiaddress | byte command | [byte_par0] | byte_address1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BYTE [0xFC] | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | BYTE [0xA5] | BYTE | BYTE | BYTE |
|  | nbyte |  |  | drive_address |  |  |  |  |  |  |  |  |

| [byte_address2] | [byte_address3] | [byte_address4] | byte_checksum |
| --- | --- | --- | --- |
| BYTE | BYTE | BYTE | BYTE |

byte_start		: 0xFC. This byte means that a command will be sent.

byte_nbyte_address	: This byte contains two information:

- drive_address	  In the first 5 bytes enter no. 31 address

- nbyte 	  The following 3 bytes (from 5 byte to 7 byte) show the bytes numbers

which will follow byte_nbyte_address before sending  byte_checksum.

byte_multiaddress	: 0xA5. This  byte indicates that command is addressed to several drives, whose

address will be specified in the following  bytes.

byte_command		: This byte represents the command (see commands protocol).

byte_par0		: The byte, which follows byte_command, represents the sent command parameter

(if necessary).

byte_address1..4		: Bytes, which follow byte_command, represent  drives addresses  to which  command

has been  addressed. Four addresses can be sent if command foresees 1 parameter. Five addresses can be sent if command does not foresee any parameter.

byte_checksum	: This byte must be calculated by the user as  complement of the less significant byte resulting from the sum of all sending bytes (including  byte_start), in order to have one byte only, as final result. Function of this byte is to verify the correctcommand transmission (see example in paragraph  6.5.5).

DRIVES ANSWER:

If  command  correct, it will be executed. If  wrong, it will be not executed, by giving no answer.

Command being addressed to several drives, they cannot answer, otherwise an hardware conflict would be caused.

EXAMPLES:

- To send RESET command to drive 0,1,2,3:

Command string: 0xFC 0xDF 0xA5 0x01 0x00 0x01 0x02 0x03 0x78		drive answer: none

- To send RESET command to drive 4,5:

Command string: 0xFC 0x9F 0xA5 0x01 0x04 0x05 0xB5			drive answer: none

6.5.3 COMMAND ADDRESSED TO ALL DRIVES:

DATA TO BE SENT TO THE DRIVES:

Commands succession  to be sent to the drives must respect following structure:

| byte_start | byte_switchall | byte_nbyte | byte command | [byte_par0] | [byte_par1] | byte_checksum |
| --- | --- | --- | --- | --- | --- | --- |
| BYTE [0xFC] | BYTE [0x00] | BYTE | BYTE | BYTE | BYTE | BYTE |

byte_start		: 0xFC. This byte means that a command will be sent.

byte_switchall		: 0x00. This byte means that a command will be sent to all drives

byte_nbyte		: This byte indicates  the bytes numbers which  will follow byte-nbyte address before

sending byte-checksum.

byte_command		: This byte represents the command (see commands protocol).

byte_par0, byte_par1	: Bytes, following byte_command, represent the sent command parameters.

byte_checksum                 : This byte must be calculated by the user as complement of the less significant byte resulting  from the sum of all sending bytes (including  byte_start), in order to have one byte only as final result.

The function of this byte is to verify the correct command transmission (see example in paragraph  6.5.5).

DRIVES ANSWER:

If  command  correct, it will be executed. If  wrong, it will be not executed, by giving no answer.

Command being addressed to several drives, they cannot reply, otherwise an hardware conflict would be  caused.

EXAMPLE:

- To send RESET command to all drives:

Command string: 0xFC 0x00 0x01 0x01 0x01		drive answer: none

6.5.4 COMMANDS PROTOCOL

| COMMAND | PARAMETERS | ANSWER | FUNCTION |
| --- | --- | --- | --- |
| 0x01 | None | byte_ack | DRIVE RESET: It stops the motor. It initializes speed and ramp to 0. |
| 0x02 | None | byte_ack | SOFTWARE START: By sending this command, motor is running according to the transmitted values (speed and ramp). |
| 0x10 | None | byte_ack + byte_start+ byte_nbyte_address+ 0xNN+byte_chksum NN=Software version | REQUEST FOR SOFTWARE VERSION |
| 0x11 | None | byte_ack | IMMEDIATE STOP: Motor slacks speed according to the preset ramp and then it stops. |
| 0x12 | None | byte_ack+byte_start+ byte_nbyte_address+byte1+byte2+byte3+byte4+byte_chksum | READING PRESENT POSITION: By receiving command 0x13 drive show present motor condition with 4 bytes. |
| 0x13 | None | byte_ack+byte_start+ byte_nbyte_address+ 0xNN+byte_chksum NN=byte, in which the 4 less significant bits represent the inputs status (1=input activated), next 2 bits represent the outputs status (1=output activated), the last 2 bits are not utilized (always at 0) | READING OF INPUTS / OUTPUTS |
| 0x14 | None | Byte_ack+Byte_start+ byte_nbyte_address+ 0xNN+Byte_chksum | INQUARY FOR DRIVE TYPE: drive signals a number corresponding to the drive type |
| 0x20 | 2 bytes, which indicate the minimum frequency (from 0 to 10000 Hz) | Byte_ack | SETTING OF MINIMUM FREQUENCY. |
| 0x21 | 2 bytes, which indicate the maximum frequency (from 0 to 10000 Hz) | byte_ack | SETTING OF MAXIMUM FREQUENCY. NOTE: If motor is running, this parameter will be acquired to next motion command. |
| 0x22 | 1 byte, which indicates the ramp inclination (from 0 to 255) expressed in ms * 10 | byte_ack | SETTING OF RAMP INCLINATION NOTE: If motor is running, this parameter will be acquired to the next motion command. |
| 0x23 | 4 bytes, which indicate the absolute motor position (expressed in 1/128 step) | byte_ack | SETTING OF HOME POSITION: Drive associates the entered value to home position of the motor |

| COMMAND | PARAMETERS | ANSWER | FUNCTION |
| --- | --- | --- | --- |
| 0x26 | 1 byte, which indicates motor resolution. If the sent byte = 0  full step mode = 1  1/2 step mode = 2  1/4 step mode = 3  1/8 step mode = 4  1/16 step mode | byte_ack | SETTING OF MOTOR RESOLUTION. NOTE: If motor is running, this parameter will be acquired to the next motion command. |
| 0x27 | 1 byte, which indicates time and mode of current reduction. The first 6 bits indicate time after which current reduction (from 0 to 63) must occur on a time basis of 32 ms. The next 2 bits indicate the reduction mode: 00 – current 0 01 - no reduction 10 – reduction to 25% 11 – reduction to 50% | byte_ack | SETTING OF ELECTRIC CURRENT REDUCTION |
| 0x28 | 1 byte, which indicates delayed answer of serial interface (from 0 to 255) expressed in µs * 512 | byte_ack | SETTING ANSWER DELAY |
| 0x29 | 1 byte: the 4 less significant bits indicate the input or the inputs, which must be enabled for STARTING (1 input enabled). The next 4 bits indicate the level of these inputs (1=active input at high level) | byte_ack | TRIGGER START (LOGIC AND): It defines the Input or the inputs and the respective Levels, which must be enabled for carrying out the START by an external command. |
| 0x2A | 1 byte: the 4 less significant bits indicate the input or the inputs, which must be enabled for STOPPING (1= enabled input). The next 4 bits indicate the level of these inputs (1=input active at high level) | byte_ack | TRIGGER STOP (LOGIC AND): It defines the input or the inputs and the respective levels, which must be enabled for carrying out the STOP by an external command. |
| 0x2B | 1 byte, indicates the level of the output ‘in position’: 0 >output motor is holding = 0 255 >output motor is holding= 1 | byte_ack | ‘IN POSITION’ OUTPUT LEVEL |
| 0x2C | 1 byte: the 4 less significant bits indicate the input or the inputs, which must be enabled for doing the HOME function (1=input enabled), the next 4 bits indicate the level of these inputs (1=input active at high level) | byte_ack | TRIGGER HOME (LOGIC AND): It defines the input or the inputs and the respective levels, which must be enabled for carrying out the HOME function by an external command. |
| 0x30 | 4 bytes, wich indicate the absolute position to reach (expressed in 1/128 step). (values admitted: from -2147483647 to 2147483647) | byte_ack | ABSOLUTE POSITIONING (RELATIVE TO THE HOME POSITION) |

| COMMAND | PARAMETERS | ANSWER | FUNCTION |
| --- | --- | --- | --- |
| 0x31 | 4 bytes, which indicate the carrying out positioning with respect to the present position of the motor (expressed in 1/128 step) (values admitted: from -2147483647 to 2147483647) | byte_ack | RELATIVE POSITIONING |
| 0x32 | 1 byte, which indicates the rotation direction if = 0 CW if = 255 CCW | byte_ack | INFINITE MOTION: When sending this command, motor is running at the speed entered in the specified rotation direction. NOTE: SEND THIS COMMAND ONLY WHEN THE MOTOR IS HOLDING. |
| 0xA0 | 5 bytes: - the 4 less significant bits indicate the input or the inputs, which must be enabled for doing the ZERO AT FLIGHT (1=input enabled), the next 4 bits indicate the level of these inputs (1=input active at high level) - next 4 bytes: these indicate the positioning to be done in the same rotation direction since when the condition expressed in the first byte occurs (values admitted: 0 to 2147483647) | byte_ack | ZERO AT FLIGHT (LOGIC AND): It defines the input or the inputs and the respective levels, which must be enabled for carrying out zeroing of the value in the present motor position, when this condition occurs, and the value to be done on occasion of this condition. |
| 0xA6 | None | byte_ack | MOTION TO ZERO VALUE |
| 0xA8 | 2 bytes, which indicate the current value (from 0 to 10000 mA) | byte_ack | CURRENT SETTING (ex. 10000 = 10A, 2000=2A). Entering a wrong value the answer will be byte_nack. |
| 0xAA | 4 bytes, which indicate the relative positioning with respect to the present position of the motor to be carried out at the next SOFTWARE START or TRIGGER START (expressed in 1/128 step). (values admitted: from -2147483647 to 2147483647) | byte_ack | SETTING OF RELATIVE VALUE (WITHOUT ANY POSITIONING) |

| COMMAND | PARAMETERS | ANSWER | FUNCTION |
| --- | --- | --- | --- |
| 0xAB | None | byte_ack+byte_start+ byte_nbyte_address+ Byte_status+byte_chksum Byte_status: BIT0: 0 =motor is holding 1 =motor is running BIT1: 0=zero at flight not active or executed 1=zero at flight active BIT2: 0=drive ok 1=drive in protection BIT3..5: input status 1,2,3 (1=activated) BIT6-7: output status 1,2 (1=activated) | DRIVE STATUS |
| 0xAC | None | Byte_status: BIT0: 0 =motor is holding 1 =motor is running BIT1: 0=zero at flight not active or executed 1=zero at flight active BIT2: 0=drive ok 1=drive in protection BIT3..5: input status 1,2,3 (1=activated) BIT6-7: output status 1,2 (1=activated) | DRIVE STATUS 1 BYTE: Only one byte Includes all information regarding the Drive status |
| 0xAD | 1 byte which shows the speed percentage (from 0 to 255) | byte_ack | PERCENTAGE SPEED CHANGE BY MOTOR MOTION: It allows to change speed by motor motion showing the relative percentage. Ex: 100%=no change, 50%=halve speed, 200%=double speed. |
| 0xAE | 4 bytes which show absolute position to join to present position (expressed in 1/128 step). (admitted value: from -2147483647 to 2147483647) | byte_ack | SETTING OF ABSOLUTE POSITION: It joins to the present position the command value. |

| COMMAND | PARAMETERS | ANSWER | FUNCTION |
| --- | --- | --- | --- |
| 0xAF | 1 byte: the less 4 significant bits show the input where zero switch will be connected (1= enabled input), next 4 bits show the level of this input (1=activated input at high lavel) | byte_ack | AXIS ZEROING: It defines the input and relative level where zero switch will be connected and it carries out axis zeroing. The zeroing phase includes: CCW motor start; the search of zero switch with rotation at max speed; stop on zero switch; disengagement of zero switch at min. speed and zeroing of absolute position. |
| 0xB0 | 1 byte: the less 4 significant bits show the input where limit switch will be connected. (1=enable input). Next 4 bits show the level of this input (1=activated input at high level). By using two switches (limit switch+ and limit switch-) they must be connected in series) | byte_ack | LIMIT SWITCH: It defines the input and the relative level where the limit switch will be connected. When the switch will be intercepted, it will cause the immediate motor stop and it allow only the opposite wise motion. The command activates this function until its disengagement through a new command with no specified input (Parameter=0). |
| 0x2A | 1 byte: the 4 less significant bits indicate the input or the inputs, which must be enabled for STOPPING (1= enabled input). The next 4 bits indicate the level of these inputs (1=input active at high level) | byte_ack | TRIGGER STOP (LOGIC OR): It defines the input or the inputs and the respective levels, which must be enabled for carrying out the STOP by an external command. |
| 0xEE | 1 byte (ammitted values: 0, 2) | byte_ack | LOW NOISE MODE: 2 – Enable 0 – Disable |

byte_ack=0x06;   byte_start=0xFC

All values preceded by ‘0x’ are hexadecimal.

NOTES:

All values sent are expressed in 1/128 step. Therefore, if we intend to carry out a revolution to a motor of 200 steps/rev.

sending value to drive would be 25600.

By changing the phase evolution mode from full step to half step, the value will remain the same.

SOME EXAMPLES OF COMMAND STRINGS ARE DESCRIBED ON THE NEXT PAGES.

6.5.5 EXAMPLES OF COMMANDS:

All examples given in the protocol refer to a drive having address 0.

| COMMAND STRING | DRIVE ANSWER | FUNCTION |
| --- | --- | --- |
| 0xFC, 0x20, 0x01, 0xE2 | 0x06 | Drive reset |
| 0xFC, 0x20, 0x02, 0xE1 | 0x06 | Software start |
| 0xFC, 0x20, 0x10, 0xD3 | 0x06, 0xFC, 0x20, 0x10 | Request for software version. The answer is 0x20 = version 2.0 |
| 0xFC, 0x20, 0x11, 0xD2 | 0x06 | Immediate stop |
| 0xFC, 0x20, 0x12, 0xD1 | 0x06, 0xFC, 0x80, 0x00, 0x00, 0x00, 0x00, 0x7D | Reading of current position. In this case the motor position is 0. |
| 0xFC, 0x20, 0x13,0xD0 | 0x06, 0xFC, 0x40, 0x22 | Input/output reading. In this case the third byte of the answer indicates that input 3 is activated. |
| 0xFC, 0x20, 0x14, 0xCF | 0x06, 0xFC, 0x20, 0x02 | Request of drive type. The involved drive has the code number 0x20. |
| 0xFC, 0x60, 0x20, 0x01, 0x5E, 0x24 | 0x06 | Setting to 350 Hz minimum frequency. |
| 0xFC, 0x60, 0x21, 0x07, 0xD0, 0xAB | 0x06 | Setting to 2000 Hz maximum frequency. |
| 0xFC, 0x40, 0x22, 0x32, 0x6F | 0x06 | Setting of ramp inclination to 50 (0.5 seconds) |
| 0xFC, 0xA0, 0x23, 0x00, 0x00, 0x00, 0x00, 0x40 | 0x06 | Setting of absolute motor position to value 0 |
| 0xFC, 0x40, 0x26, 0x00, 0x9D | 0x06 | Setting of motor resolution to full steps |
| 0xFC, 0x40, 0x27, 0x99, 0x03 | 0x06 | Setting of current reduction to 25% of rated current after a time of 25 (25x32ms=0.8 seconds) |
| 0xFC, 0x40, 0x28, 0x03, 0x98 | 0x06 | Setting answer delay of the serial interface (3x512s) |
| 0xFC, 0x40, 0x29, 0x44, 0x56 | 0x06 | Setting of start trigger on up-front input 3 (signal transition from low to high) |
| 0xFC, 0x40, 0x2A, 0x22, 0x77 | 0x06 | Setting of trigger stop on up-front input 2 (signal transition from low to high) |
| 0xFC, 0x20, 0x2B, 0x00, 0xB8 | 0x06 | Output level in position 0 when motor Is holding |
| 0xFC, 0x40, 0x2C, 0x11, 0x86 | 0x06 | Setting of home trigger of up-front input 1 (signal transition from low to high) |
| 0xFC, 0xA0, 0x30, 0x00, 0x00, 0x64m 0x00, 0xCF | 0x06 | Absolute positioning equal to 1 motor rev. (value expressed in 1/128 of a step = 25600) |
| 0xFC, 0xA0, 0x31, 0x00, 0x00, 0x64, 0x00, 0xCE | 0x06 | Relative positioning regarding present position equal to 1 motor rev. CW (value expressed in 1/128 step = 25600) |

| COMMAND STRING | DRIVE ANSWER | FUNCTION |
| --- | --- | --- |
| 0xFC, 0xA0, 0x31, 0xFF, 0xFF, 0x64, 0x00, 0xCE | 0x06 | Relative positioning regarding present position equal to 1 motor rev. CCW (value expressed in 1/128 step = -25600) |
| 0xFC, 0x40, 0x32, 0x00, 0x91 | 0x06 | Infinite CW motion |
| 0xFC, 0x40, 0x32, 0xFF, 0x92 | 0x06 | Infinite CCW motion. |
| 0xFC, 0xC0, 0xA0, 0x11, 0x00, 0x00, 0x64, 0x00, 0x2E | 0x06 | Zero at flight active on input 1, transition low/high, with value to be carried out by the activation of the input equal to 1 motor rev.(expressed in 1/128 of a step = 25600) |
| 0xFC, 0x20, 0xA6, 0x3D | 0x06 | Motion to value zero |
| 0xFC, 0x60, 0xA8, 0x19, 0x64, 0x7E | 0x06 | Setting current to 6.5A |
| 0xFC, 0xA0, 0xAA, 0xFF, 0xFF, 0x64, 0x00, 0x57 | 0x06 | Setting of relative value to be carried out at the next START (value expressed in 1/128 of a step = -25600) |

EXAMPLE OF CALCULATION  Byte_Checksum (last byte of the string):

For sending the reset command to drive 0, the string will be as follows:

0xFC, 0x20, 0x01, Byte_Checksum.

For calculating the last byte, proceed as follows:

- Sum up all bytes of the command: 0xFC + 0x20 + 0x01 = 0x11D

- Consider only the less significant byte: 1D

- Complement the byte found, so to obtain the ByteChecksum: 0xFF – 0x1D = E2

The complete command to be sent will be as follows:

0xFC, 0x20, 0x01, 0xE2

6.5.6 SIMPLE MOTION PROGRAM:

This is a simple sequence of instructions to command the APS in SERIAL MODE:

The drive used is set to address 0

FUNCTION				COMMAND STRING				DRIVE ANSWER

Reset driver				0xFC 0x20 0x01 0xE2 				0x06

Set serial answer delay to 5 msec		0xFC 0x40 0x28 0x0A 0x91			0x06

Set Fmin to  450 Hz			0xFC 0x60 0x20 0x01 0xC2 0xC0  		0x06

Set Fmax to  5000 KHz			0xFC 0x60 0x21 0x13 0x88 0xE7			0x06

Set ramp to   100 msec 			0xFC 0x40 0x22 0x0A 0x97			0x06

Set step division to ½ step			0xFC 0x40 0x26 0x01 0x9C			0x06

Start relative quote 10 revolution CW	0xFC 0xA0 0x31 0x00 0x03 0xE8 0x00 0x47	0x06