OVERVIEW

This sub-directory contains a demonstration program for use

with either one analog input or analog output module. The

source code for this program was written using Microsoft

Visual Basic Professional Edition version 3.0 and may be

distributed freely. This source code may also be modified,

or customized, to build a more complete application program.

The supplied source code is written to maintain up to four

open serial ports (COM1:..COM4:). This code also contains

routines to read and write analog or digital data values to

any module. Upon examining the source code users will find

four different read data functions (RD1..RD4), analog output

subroutines (AO1..AO4), digital input functions (DI1..DI4),

read events functions (RE1..RE4) and digital output

subroutines (DO1..DO4). The numbers 1..4 associated with

each routine indicate the port value thru which module

commands will be transmitted and responses received.

Each of the prewritten functions and subroutines utilize two

routines that actually transmit and receive data. The

prewritten routines mentioned above simply format output

command strings and properly parse module response messages.

The lower-level routines that transmit and receive data are

transmit procedures TX1..TX4 and string receive functions

RX1..RX4. The numeric values associated with each routine

again determines the correct communications port value. The

transmit procedures will write any string of characters

(passed to it) to the proper communications port. The

transmit procedures can also be called by any user written

routines.

The receive data functions RX1..RX4 each return strings of

characters from a communications port. The receive functions

each have an Interval Timer associated them. The timers are

used to provide a graceful ending to each Receive function

if no data is returned from a module. Once a complete module

command string is transmitted, the receive function can be

called. The timer is started first in the receive function.

The receive function then continuously tests the

communications port for any received characters. If no

characters have been received when the interval timer

expires, the receive function will return a device timeout

message ("TIMEOUT"). The device timeout message should only

occur if an incorrect module address is transmitted or the

module is not properly powered up or connected. The Timer

interval is specified in the Communications Port settings

screen.

The response messages received from every module each

contain a fixed first character and are all terminated by a

carriage return. The first character of each message can be

either a "*" or "?". The "*" indicates the beginning of a

valid response and the "?" indicates an error occurred. In

order to properly frame each module response, this source

code will wait for either the "*" or "?" characters and then

read all incoming characters up to a carriage return. Any

leading characters that are received before the "*" or "?"

will be flushed from the receive buffer and disgarded. This

technique will ensure that only the response data is

returned back to the program and eliminates any software

parsing problems that can arrise when communicating with

daisy-chained RS-232 modules.

DEMONSTRATION

This demonstration program is written for the primary reason

of supplying the prewritten communications handling

subroutines and functions for use with the analog and

digital I/O products. These routines can be used as a base,

or starting point, towards developing a more safisticated

applications program and can be located in the source file

COMM.BAS.

COMMUNICATIONS SETTINGS

The demonstration program contains four functional screens.

Each screen is selected from the program main menu. The

first screen that must be called is the Communications

Settings. Select "Setup" from the main menu to configure and

open a serial communciations port. A port must be opened

before the demonstration screens can be run.

The three remaining screens are functional screens that can be

used to demonstrate the features of either an analog input,

analog output or digital I/O module. These screens can be

viewed by selecting "Demo" from the main menu. Select proper

screen depending on which type of module is connected.

Note:	As previously mentioned, this program can support

four open communications ports simultaneously.

Therefore, if multiple ports  are being used, any

analog I/O module connected to ports 1 thru 4

can communicate with this program. Use the

"SETUP" button on either demonstration screen

to specify the correct port value and module address.

ANALOG INPUT

The Analog Input screen displays the present analog input,

events counter and digital input data values. The analog and

event counter data values are displayed both in text and

bargraph formats. The digital input status values are

displayed graphically using bitmaps which represent

electrical LED's. By simply connecting each input bit to the

module "GND" terminal, the proper display LED should change

color shade. The digital outputs can be switched on and off

by clicking on any of eight displayed switches. Each switch

corresponds to an individual digital output bit on the

connected module.

Once the Analog Input screen is displayed, use the mouse to

depress the "Setup" button. Specify the correct module

address and communications port value. The address must be

entered using decimal equivalent of the ASCII character

address. For example, enter 49 if the module address is an

ASCII character "1". Proper communications port values are 1

thru 4. Graph minimum and maximum values can be entered as

desired or necessary. Return to the Analog Input

demonstration screen when entries are completed.

Start the analog input demonstration by pressing the "Run"

button. To stop the demonstration depress the "Stop" button.

To return to the main menu, press the "Cancel" button.

ANALOG OUTPUT

The Analog Output screen will allow a user to enter analog

data values. The analog data values will be formatted by

this program and output to the connected module. Analog

output values can also be generated using the slide bar

control located on the demonstration screen. Anytime the

slide control is moved, the new control data value will be

formatted and transmitted. Use the "Setup" screen to specify

the minimum and maximum slide bar values.

The Analog Output screen also displays the present status of

three digital input bits located on each analog output

module. The digital input status values are displayed

graphically using bitmaps which represent electrical LED's.

By simply connecting each input bit to the module "GND"

terminal, the proper display LED should change color shade.

Once the Analog Output screen is displayed, use the mouse to

depress the "Setup" button. Specify the correct module

address and communications port value. The address must be

entered using decimal equivalent of the ASCII character

address. For example, enter 49 if the module address is an

ASCII character "1". Proper communications port values are 1

thru 4. Analog output slide bar minimum and maximum values

can be entered as desired or necessary. Return to the Analog

Output demonstration screen when entries are completed.

Start the analog output demonstration by pressing the "Run"

button. To stop the demonstration depress the "Stop" button.

To return to the main menu, press the "Cancel" button.

DIGITAL INPUT/OUTPUT

The Digital Input/Output screen will allow a user to display

digital data values. The digital input status values are

displayed graphically using bitmaps which represent

electrical LED's. By simply connecting each input bit to the

module "GND" terminal, the proper display LED should change

color shade.

The digital outputs can be switched on and off by clicking

on any of eight displayed switches. Each switch corresponds

to an individual digital output bit on the connected module.

Depress the "Setup" button to specify the correct module

address and communications port value. The address must be

entered using decimal equivalent of the ASCII character

address. For example, enter 49 if the module address is an

ASCII character "1". Proper communications port values are 1

thru 4.

Start the demonstration by pressing the "Run" button. To

stop the demonstration depress the "Stop" button. To return

to the main menu, press the "Cancel" button.