tam

Application Note

TDS Screen Capture

via the RS-232 option

This screen image was captured from a TDS oscilloscope over the RS-232 port using the Windows Terminal program.

To transfer the screen image follow the procedure outlined below.

1. From the TDS Utility IO menu set the  port to RS-232.

Harware Setup:  9600,1,none,on.

Software Setup: Soft Flagging Off.

3. Set the hard copy format. ( This image was produced using PCX )

2. Set the communications settings in Windows Terminal to match those of the oscilloscope. 9600,8,1,None, Hardware.

4. In the Terminal program, select Receive Text File (under the Transfers Menu) and check the Save Controls . This will enable the Terminal program to save binary screen images.

5. Once you have the image you want on the oscilloscope display, press the HARD COPY button.

6. When the transfer is complete, click on the STOP button in the Terminal program. Now you have a screen image you can import into Microsoft Word for Windows.

7. In Word for Windows, select INSERT PICTURE and enter the filename of the screen image. The file will be converted and displayed in your document. If you used the EPS hard copy format the screen image will not appear in your document, but an EPS block will appear. When the document is printed the screen image will be printed.

8. The following diagrams should be used to build the appropriate cable.

µ §

µ §