Polarized 3He Pre Beam Checklist Date _____time _______

Last revised/6/11/01

#### This checklist will be performed after every restricted access to Hall A that maintenance is performed

#### Person Responsible for checklist ___________________

People checking list _______________________________________________________

###### Left Arm (Old Electron)

##### Spectrometers

___	correct angle ___________ (not to be used for calculations)

___	correct pointing _________(not to be used for calculations)

___	Collimator operation at 3 positions

___	check camera angle for movement in both directions

___	Check spectrometer for obstructions to movement

___	Check intergen bottles for correct pressure

___	Insure that Intergen alarm switch is in the normal position

___	Insure that 15-degree stop pin is installed

___	Insure that outer limit stop is installed (if used)

___	Maximum/minimum angles for spectrometer _______to_______

Vacuum

___	Insure that turbo backing pump is on

___	Turbo on at turbo controller in rack # 1H71B01

___	Pump valves open at valve controller in rack # 1H71B01  channel #2

___	Convectron gages read “0” millitorr

___	insure roughing pump at dipole entrance has sufficient oil

___	Cold cathode gages on at gage in rack # 1H71B01 cold cathode < 5x10-5

___	Actual cold cathode reading ________

___	Entrance & exit vacuum windows functional

###### Left Arm (Old Electron)

Magnet controls

Q1

___	Q1 full of liquid  (60%)

___	open lead flows on Q1 to 80 slm as read from rack #Q171Q

___	actual lead flows	A______	B______

___	cctv camera on and focused

D1

___	Dipole full of liquid  (80%)

___	open lead flows on Dipole to 80 slm as read from rack #D171Q

___	actual lead flows	A______	B______

Q2

___	Q2 full of liquid  (80%)

___	open lead flows on Q2 to 60 slm as read from the Q2 instrument rack meter.

___	actual lead flows	1______	2______

___	Insure that lead heaters are on and operating

Q3

___	Q3 full of liquid  (80%)

___	open lead flows on Q3 to 60 slm as read from the Q3 instrument rack meter.

___	actual lead flows	1______	2______

___	Insure that lead heaters are on and operating

Left Arm (Old Electron)

Power supplies

POWER SUPPLY TURN ON PROCEDURES

___	Verify UPSs as operational on all power supply controls (with no current on magnets only)

___	red rotating beacons on

Q1:

___	visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts ,etc.)

___	unlock power disconnect switch and turn on AC power

___	visually check power supply for faults

___	when all faults have been cleared, insure that power supply is in remote control

Q2:

___	visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts ,etc.)

___	unlock power disconnect switch and turn on AC power

___	turn on both sets of three pole breakers located on power supply

___	visually check power supply for faults

___	when all faults have been cleared, lift lever on lower right side of supply

___	insure that power supply is in remote control

Q3:

___	visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts ,etc.)

___	unlock power disconnect switch and turn on AC power

___	turn on both sets of three pole breakers located on power supply

___	visually check power supply for faults

___	when all faults have been cleared, lift lever on lower right side of supply

___	insure that power supply is in remote control

Dipole:

___	Visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts, etc.)

___	Unlock power disconnect switch and turn on AC power

___	Turn on power lever on right upper side of supply

___	Visually check power supply for faults on supply and at rack #

___	When all faults have been cleared, insure that power supply is in remote control

___	Cctv camera on and focused

___	Check power supply for proper polarity 	positive____	negative____

___	NMR gradient compensation for on and proper polarity

positive____ negative____

###### Right Arm (Old Hadron)

##### Spectrometers

___	correct angle ___________ (not to be used for calculations)

___	correct pointing _________(not to be used for calculations)

___	Collimator operation at 3 positions

___	check camera angle for movement in both directions

___	Check spectrometer for obstructions to movement

___	Check intergen bottles for correct pressure

___	Insure that Intergen alarm switch is in the normal position

___	Insure that 15-degree stop pin is installed

___	Maximum/minimum angles for spectrometer ___45__to_______

Vacuum

___	Insure that turbo backing pump is on

___	Turbo on at turbo controller in rack # 1H71B01

___	Pump valves open at valve controller in rack # 1H71B01  channel #2

___	Convectron gages read “0” millitorr

___	insure roughing pump at dipole entrance has sufficient oil

___	Cold cathode gages on at gage in rack # 1H71B01 cold cathode < 5x10-5

___	Actual cold cathode reading ________

___	Check dipole turbo for on

###### Right Arm (Old Hadron)

Magnet controls

Q1

___	Q1 full of liquid  (60%)

___	open lead flows on Q1 to 80 slm as read from rack #Q172Q

___	actual lead flows	A______	B______

___	cctv camera on and focused

D1

___	Dipole full of liquid  (80%)

___	open lead flows on Dipole to 80 slm as read from rack #D172Q

___	actual lead flows	A______	B______

Q2

___	Q2 full of liquid  (80%)

___	open lead flows on Q2 to 60 slm as read from the Q2 instrument rack meter.

___	actual lead flows	1______	2______

___	Insure that lead heaters are on and operating

Q3

___	Q3 full of liquid  (80%)

___	open lead flows on Q3 to 60 slm as read from the Q3 instrument rack meter.

___	actual lead flows	1______	2______

___	Insure that lead heaters are on and operating

###### Right Arm (Old Hadron)

Power supplies

POWER SUPPLY TURN ON PROCEDURES

___	Verify UPSs as operational on all power supply controls (with no current on magnets only)

___	Red rotating beacons on

Q1:

___	Visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts, etc.)

___	Unlock power disconnect switch and turn on AC power

___	Visually check power supply for faults

___	When all faults have been cleared, insure that power supply is in remote control

Q2:

___	Visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts ,etc.)

___	Unlock power disconnect switch and turn on AC power.

___	Turn on both sets of three pole breakers located on power supply.

___	Visually check power supply for faults.

___	When all faults have been cleared, lift lever on lower right side of supply.

___	Insure that power supply is in remote control.

Q3:

___	Visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts, etc.)

___	Unlock power disconnect switch and turn on AC power

___	Turn on both sets of three pole breakers located on power supply

___	Visually check power supply for faults

___	When all faults have been cleared, lift lever on lower right side of supply

___	Insure that power supply is in remote control

Dipole:

___	Visual inspection of main current leads, dump resistor, and lead flags (for condition, visual shorts, etc.)

___	Unlock power disconnect switch and turn on AC power

___	Turn on power lever on right upper side of supply.

___	Visually, check power supply for faults on supply and at rack #OD172Q.

___	When all faults have been cleared, insure that power supply is in remote control.

___	Cctv camera on and focused

___	Check power supply for proper polarity 	positive____	negative____

___	NMR gradient compensation for on and proper polarity

Positive____	negative____

Target

___	Insure that all window shielding is installed

___	Inspect laser enclosures for possible light leaks and note any anomalies

___      Drain He3 air compressor at tank and separator

___	Insure that target ventilation system is on, and not overheating

___	Insure that vent system throttle valve is in the “operate“ range

___	Insure that the oven gas per heater variac is set at 70

Exit beam tube

___	Diffuser cooler on

___	Diffuser water level ok

___	Inspect entrance window and note any anomalies __________________________

___	Backing  pump “on” at pump and operational

___	Valve “open” (temp manual at pump)

___	Turbo “on” at rack #  1H75B09

___	Gages operational

___	Convectron “<5” millitorr at rack # 1H75B09

___	Actual  convectron reading ________

Entrance beam tube

___	Insure that Beam line turbo is on and running

___	Insure that there is cooling water flow to the Moeller Dipole

___	Insure that EP turbo is on and running

___	Inspect exit window and note any anomalies __________________________

___	Instrument air compressor functioning normally

___	Give operational control of beam line valves to MCC

Call MCC and say “I am turning the control key from maintenance to operational are you ready” after they say yes, turn key and tell them

“You have Valve control could you please open the valves and make an e-log entry”

___	 Beam line vacuum valves “open” (visually checked)

## Hall

___	All interlocks in rack # 1H75B08 indicate green except target entrance and exit valves

___	Check 3 Moeller power supplies for on (usually remote)

___	Insure installation of Ion chambers at EP, target, and 2 at beam dump

___	Correct LCW flow and pressure (100 psi supply and 60 psi return)

___	Cctv cameras on and focused

___	Cctv monitors at X terminal off

___	Clear of unnecessary equipment

___	Man lift and Forklift in truck ramp.

___	Insure that all lifting slings and safety harnesses are correctly stored and that the storage cage is at least 90 deg from the beam dump and at least 60 ft from the target

___	Perform pre sweep of run safe boxes.

___	Unnecessary personnel exit Hall.

___	Move Electron spectrometer stairs clear of lower balcony.

___	Insure polar crane is positioned over the entrance beam pipe,

and that power is off at the power disconnect switch

___	Insure that spectrometer entrance window guards are removed

___	Insure that spectrometer exit window guards are removed

___	Deliver checklist to shift leader or run coordinator

Name of person checklist was delivered to ______________________