VMIVME 3128

BASE 0/X = RIGHT SEPTUM                                           BASE 1/X = LEFT SEPTUM

ANALOG TO DIGITAL (96 PIN)

| BASE/SLOT | TERM # | EXT. TERM | CHANNEL | BLOCK # 1 | SIGNAL DESCRIPTION | P4 PIN |
| --- | --- | --- | --- | --- | --- | --- |
| 0/1 | 1(+) | 1 | 0 | 1 / A1 | MAGNET VOLTAGE (FROM MAGNET) | 01A |
| 0/1 | 1(-) | 2 | 0 | 1 /C1 | MAGNET VOLTAGE (FROM MAGNET) | 01C |
| 0/1 | 2(+) | 3 | 1 | 1 / A2 | TRANSITION LEAD 1 | 02A |
| 0/1 | 2(-) | 4 | 1 | 1 / C2 | TRANSITION LEAD 1 | 02C |
| 0/1 | 3(+) | 5 | 2 | 1 / A3 | TRANSITION LEAD 2 | 03A |
| 0/1 | 3(-) | 6 | 2 | 1 / C3 | TRANSITION LEAD 2 | 03C |
| 0/1 | 4(+) | 7 | 3 | 1 / A4 | POWER SUPPLY VOLTAGE | 04A |
| 0/1 | 4(-) | 8 | 3 | 1 / C4 | POWER SUPPLY VOLTAGE | 04C |
| 0/1 | 5(+) | 9 | 4 | 1 / A5 | POWER SUPPLY CURRENT | 05A |
| 0/1 | 5(-) | 10 | 4 | 1 / C5 | POWER SUPPLY CURRENT | 05C |
| 0/1 | 6(+) | 11 | 5 | 1 / A6 | MAGNET CURRENT (SHUNT) | 06A |
| 0/1 | 6(-) | 12 | 5 | 1 / C6 | MAGNET CURRENT (SHUNT) | 06C |
| 0/1 | 7(+) | 13 | 6 | 1 / A7 | DUMP RESISTER V0LTAGE | 07A |
| 0/1 | 7(-) | 14 | 6 | 1 / C7 | DUMP RESISTER VOLTAGE | 07C |
| 0/2 | 1(+) | 1 | HE FLOW METER | CH1 2(+) | + He LEAD FLOW GAS |  |
| 0/2 | 1(-) | 2 | HE FLOW METER | CH1 11(-) | + He LEAD FLOW GAS READBACK |  |
| MASS FLOW METER | EXTERN | 3(+) | 7 & (ISO 11) | 1 / A8 & CRYO | + He LEAD FLOW GAS | 08A |
| MASS FLOW METER | EXTERN | 8(-) | 7 & (ISO 11) | 1 / C8 & CRYO | + He LEAD FLOW GAS | 08C |
| 0/2 | 2(+) | 3 | HE FLOW METER | CH2 2(+) | - He LEAD FLOW GAS READBACK |  |
| 0/2 | 2(-) | 4 | HE FLOW METER | CH2 11(-) | - He LEAD FLOW GAS READBACK |  |
| MASS FLOW METER | EXTERN | 4(+) | 8 & (ISO 12) | 1 / A9 & CRYO | - He LEAD FLOW GAS | 09A |
| MASS FLOW METER | EXTERN | 9(-) | 8 & (ISO 12) | 1 / C9 & CRYO | - He LEAD FLOW GAS | 09C |
| CONVECTRON | MINI PHONO | XXX | 9 | 1 / A10 | VESSEL TC GUAGE | 10A |
| CONVECTRON | MINI PHONO | XXX | 9 | 1 / C10 | VESSEL TC GUAGE | 10C |
| 0/2 | 5(+) | 9 | 10 | 1 / A11 | LIQUID N2 PRESSURE | 11A |
| 0/2 | 5(-) | 10 | 10 | 1 / C11 | LIQUID N2 PRESSURE | 11C |
| 0/2 | 7(+) | 13 | 11 | 1 / A12 | LIQUID He PRESSURE | 12A |
| 0/2 | 7(-) | 14 | 11 | 1 / C12 | LIQUID He PRESSURE | 12C |
| 0/2 | 8(+) | 15 | 12 | 1 / A13 | He LIQUID LEVEL | 13A |
| 0/2 | 8(+) | 15 | N/A | ISO 10 | He LIQUID LEVEL | N/A |
| 0/2 | 8(-) | 16 | 12 | 1 / C13 | He LIQUID LEVEL | 13C |
| 0/2 | 8(-) | 16 | N/A | ISO 10 | He LIQUID LEVEL | N/A |
| 0/2 | 6(+) | 11 | 13 | 1 / A14 | N2 LIQUID LEVEL | 14A |
| 0/2 | 6(+) | 11 | N/A | ISO 13 | N2 LIQUID LEVEL | N/A |
| 0/2 | 6(-) | 12 | 13 | 1 / C14 | N2 LIQUID LEVEL | 14C |
| 0/2 | 6(-) | 12 | N/A | ISO 13 | N2 LIQUID LEVEL | N/A |
| 1/1 | 1(+) | 1 | 14 | 1 / A15 | MAGNET VOLTAGE (FROM MAGNET) | 15A |
| 1/1 | 1(-) | 2 | 14 | 1 / C15 | MAGNET VOLTAGE (FROM MAGNET) | 15C |
| 1/1 | 2(+) | 3 | 15 | 1 / A16 | TRANSITION LEAD 1 | 16A |
| 1/1 | 2(-) | 4 | 15 | 1 / C16 | TRANSITION LEAD 1 | 16C |
| 1/1 | 3(+) | 5 | 16 | 1 / A17 | TRANSITION LEAD 2 | 17A |
| 1/1 | 3(-) | 6 | 16 | 1 / C17 | TRANSITION LEAD 2 | 17C |
| 1/1 | 4(+) | 7 | 17 | 1 / A18 | POWER SUPPLY VOLTAGE | 18A |
| 1/1 | 4(-) | 8 | 17 | 1 / C18 | POWER SUPPLY VOLTAGE | 18C |
| 1/1 | 5(+) | 9 | 18 | 1 / A19 | POWER SUPPLY CURRENT | 19A |
| 1/1 | 5(-) | 10 | 18 | 1 / C19 | POWER SUPPLY CURRENT | 19C |
| 1/1 | 6(+) | 11 | 19 | 1 / A20 | MAGNET CURRENT (SHUNT) | 20A |
| 1/1 | 6(-) | 12 | 19 | 1 / C20 | MAGNET CURRENT (SHUNT) | 20C |
| 1/1 | 7(+) | 13 | 20 | 1 / A21 | DUMP RESISTER VOLTAGE | 21A |
| 1/1 | 7(-) | 14 | 20 | 1 / C21 | DUMP RESISTER VOLTAGE | 21C |
| 1/2 | 1(+) | 1 | HE FLOW METER | CH1 2(+) | + He LEAD FLOW GAS |  |
| 1/2 | 1(-) | 2 | HE FLOW METER | CH1 11(-) | + He LEAD FLOW GAS |  |
| MASS FLOW METER | EXTERN | 3(+) | 21 & (ISO 3) | 1 / A22 & (CRYO) | + He LEAD FLOW GAS |  |
| MASS FLOW METER | EXTERN | 8(-) | 21 & (ISO 3) | 1 / C22 & (CRYO) | + He LEAD FLOW GAS |  |
| 1/2 | 2(+) | 3 | HE FLOW METER | CH2 2(+) | - He LEAD FLOW GAS |  |
| 1/2 | 2(-) | 4 | HE FLOW METER | CH2 11(-) | - He LEAD FLOW GAS |  |
| MASS FLOW METER | EXTERN | 4(+) | 22 & (ISO 4) | 1 / A23 & (CRYO) | - He LEAD FLOW GAS |  |
| MASS FLOW METER | EXTERN | 9(-) | 22 & (ISO 4) | 1 / C23 & (CRYO) | - He LEAD FLOW GAS |  |
| CONVECTRON | MINI PHONO | XXX | 23 | 1 / A24 | VESSEL TC GUAGE | 24A |
| CONVECTRON | MINI PHONO | XXX | 23 | 1 / C24 | VESSEL TC GUAGE | 24C |
| 1/2 | 5(+) | 9 | 24 | 1 / A25 | LIQUID N2 PRESSURE | 25A |
| 1/2 | 5(-) | 10 | 24 | 1 / C25 | LIQUID N2 PRESSURE | 25C |
| 1/2 | 7(+) | 13 | 25 | 1 / A26 | LIQUID He PRESSURE | 26A |
| 1/2 | 7(-) | 14 | 25 | 1 / C26 | LIQUID He PRESSURE | 26C |
| 1/2 | 8(+) | 15 | 26 | 1 / A27 | He LIQUID LEVEL | 27A |
| 1/2 | 8(+) | 15 | N/A | ISO 2 | He LIQUID LEVEL | N/A |
| 1/2 | 8(-) | 16 | 26 | 1 / C27 | He LIQUID LEVEL | 27C |
| 1/2 | 8(-) | 16 | N/A | ISO 2 | He LIQUID LEVEL | N/A |
| 1/2 | 6(+) | 11 | 27 | 1 / A28 | N2 LIQUID LEVEL | 28A |
| 1/2 | 6(+) | 11 | N/A | ISO 1 | N2 LIQUID LEVEL | N/A |
| 1/2 | 6(-) | 12 | 27 | 1 / C28 | N2 LIQUID LEVEL | 28C |
| 1/2 | 6(-) | 12 | N/A | ISO 1 | N2 LIQUID LEVEL | N/A |
| SWEEPER MAGNET P/S (J9) | 7(+) | RED | 28 | 1/A29 | CURRENT READBACK | 29A |
| SWEEPER MAGNET P/S (J9) | 15(-) | ORNG. | 28 | 1/C29 | CURRENT READBACK | 29C |
| SWEEPER MAGNET P/S (J9) | 12(+) | YEL. | 29 | 1/A30 | VOLTAGE READBACK | 30A |
| SWEEPER MAGNET P/S (J9) | 13(-) | GRN. | 29 | 1/C30 | VOLTAGE READBACK | 30C |
| 3V POWER SUPPLY | + | RED | 30 | 1/A31 | P/S VOLTAGE SET READBACK |  |
| 3V POWER SUPPLY | - | BLK | 30 | 1/C31 | P/S VOLTAGE SET READBACK |  |
| 3V POWER SUPPLY | + | RED | 31 | 1/A32 | P/S VOLTAGE SET READBACK |  |
| 3V POWER SUPPLY | - | BLK | 31 | 1/C32 | P/S VOLTAGE SET READBACK |  |