Task Hazard Analysis Worksheet

#A-08-106-T										Date 6/2/08

| Complete all information. Use as many sheets as necessary. |  |
| --- | --- |
| Task location Hall A | Task title General Survey Activities |
| Division Physics | Prepared by folts |
| Department Hall A | Reviewed by |
| Supervisor | Approved by C. Curtis |
| Standard Requirements: EH&S manual, chapters 6110, 6120, 6131, 6132, 6146, 6410, 6440, 6620. Equipment transportation procedure. Van ramp procedure. |  |

| Task | Potential Hazards | Risk Code before mitigation | Safe Procedures/Practices/Controls | Risk Code after mitigation |
| --- | --- | --- | --- | --- |
| - Transport equipment to job site. 2. Assess the work area for hazards. 3. Set up equipment and targets. 4. Measure survey monuments and targets on component. 5. Adjust any components that are out of tolerance. 6. Complete task 7. Transport equipment back to survey lab | Back strain General area hazards include radiation, ODH, LOTO issues. Trip hazards. Falls. Class II laser usage for Tracker and Mekometer. Awkward positions, restricted space. No additional hazards No additional hazards | 2 1 1 1 1 1 2 | Proper lifting techniques; use of van ramps (see procedures). Avoid lifting when possible (transfer equipment under beamline etc.) or use mechanical aids (carts, transfer table in FEL etc.) Training required; be aware of signs denoting radiation areas etc., and others working in the area. Define energized state of components to be worked on (EH&S 6110, 6440) Set up equipment and power cords so as to avoid trip hazards (EH&S 6131). Set up cones if targets are left on the floor. Use proper techniques for ladders (EH&S 6132), or fall restraint, if required (EH&S 6131-T2) Set up laser warning signs. (EH&S 6410). Use PPE (e.g. gloves, mats) if necessary (EH&S 6620). Use correct tools (EH&S 6120) Break down equipment in proper order. Account for all tools and equipment. Leave the worksite in an orderly state. As step 1 above | 0 0 0 0 0 0 0 |

When a completed analysis indicates that the estimated risk code for any of the steps of this task is “medium” or higher (RC≥3), then develop a formal written procedure for the task and have it reviewed and approved prior to beginning the work.