List of materials

New diffuser (with indium seal installed and leak checked)

New stainless steel bolts and nuts

Tyvek suits

Tyvek caps

Yellow booties	radcon supplied

Yellow covers		radcon supplied

Dosimeters		radcon supplied

Respirators 		if necessary

Blue latex gloves

Misc. tools 		use and loose

Three Hall A techs 	2 inside 1 outside

Procedure

Bleed up the exit beam pipe vacuum system to atmosphere

Perform confined space check

Perform radcon pre-entry checks

Perform Be contamination checks if necessary

Dress out as per Radcon RWP	2 techs

Drain water from diffuser water system

Gravity drain as much water as possible then pressurize system with gas until system is relatively empty.  While the system is empty change the filter in manifold.

Remove old diffuser

Cover the Be window with duct tape to prevent the spread of any contamination.

Remove 8 - ½ inch swaglocks from diffuser.

Remove bolts holding diffuser to exit beam pipe leaving 2 bolts in place that are loose to hold the weight of the diffuser.

Using a puddy knife and a hammer, separate the diffuser from the exit beam pipe flange.

While holding the weight of the flange, remove the bolts and lower the diffuser to the floor.

Install duct tape over the inner Be window to prevent any contamination

Using a wooden stick and indium remove any remaining indium from the exit beam pipe flange and clean the flange with Isopropyl and inspect flange to insure integrity of the sealing surface.

Move old diffuser and hardware from the working area as specified by radcon.

Install new diffuser

Position the new diffuser below the exit beam pipe flange with the indium seal clean and in the up position.

While holding the new diffuser away from the flange, loosely install 2  new bolts  through the flange and diffuser for alignment.

With one tech holding the weight of the diffuser and holding it away from the flange, install the remaining new bolts.

In one motion, move the diffuser against the flange and hold it while the bolts are tightened incrementally to insure even pressure on the indium flange using an opposing torque pattern to 45 ft lbs in no more than 5 lb steps.

Install the 8 - ½ in swaglocks onto the new diffuser.

Insure that the manifold filter is installed and that all fittings are tight.

Pressurize the water system to 60 lbs and leak check the swaglock fittings and filter fittings with snoop.

Start the water circulating system and insure that there is at least .5 GPM through the flow meter and that there are no leaks in the manifold or diffuser.

Exit beam dump as per RWP.

Dress out as per RWP

Wait a minimum of 4 hours (24 hours is preferable) and re-torque the indium seal to 45 ft lbs

Leak check exit beam pipe at diffuser indium seal

Exit beam dump as per RWP