1. Define the scope of work

Install the Big Bite Spectrometer arm on the pivot

2. Analyze Hazard

| Task | Potential Hazards | Risk Code before mitigation | Safe Procedures/Practices/Controls | Risk Code after mitigation |
| --- | --- | --- | --- | --- |
| Installing the Big Bite spectrometer | Crush, bump, pinch Drop load Damage to cables and air lines | 3 1 | Equipment specific training Proper PPE, crane training | 1 0 |

3. Develop and implement Hazard controls

Follow procedure and THA

4. Perform work within controls (follow procedure)

- Summary of job

Install the Big Bite Spectrometer arm on the pivot

- Affected systems

Area where the arm is to be installed

Crane

- List of PPE

Steel toed shoes

Gloves

Hard hat

- Identify danger zones

Area where the arm is to be installed

Area above where the arm is to be installed

- Number of people needed

2

- List of materials

Proper length slings and rigging

Crane

Forklift

Big Bite stand (Arm)

Big bite stand extensions

Assorted hardware

- Procedure

The big bite stand weighs approximately 3500 lbs including the stabilizing legs. We have used 12 ft slings with success.

Using the CG marks on the sides of the stand rig the stand for a 2 point pick to the centerline of the stand.

Using a shackle connect at least 24 feet of lings from the connection point to the hook in order to cleat the cryo lines. 2 -12 foot slings work well for getting the sling over the cryo lines.

With the Big Bite stand staged in position for installation rotate the stand 90 degrees and unbolt the stabilizer legs from the pivot end

Fine tune the pick points and rotate the stand back to the original installation position.

Move the stand as close to the pivot as the cryo lines will allow and stabilize it using cribbing if necessary

Re-rig the sling to the center of the cross over bar on the pivot end of the stand using the same length sling as before. We used 3-12 ft lengths and they worked well.

Position the small forklift forks under the cross over bar on the end of the stand away from the pivot

Being careful of the cables and utilities under the balcony and exit beam pipe support, use the crane and the forklift in unison, position the stand’s Hillman rollers on their perspective rails and lower them into position.

Remove the sling and move the forklift clear of the Big Bite stand.

Rotate the Big Bite stand to the required installation position.

Move the previously disconnected stabilizer legs 4 feet behind the Big Bite stand

Attach the installation extensions to the Big bite stand and the stabilizer legs.

The stand is now ready for the installation of the magnet and detector assembly.

- Post job briefing and cleanup

5. Provide feedback and continuous improvement. (Document any mistakes, corrections, changes and pictures and implement changes.)