Define the scope of work

This procedure will define removal and/or loading of a detector panel and its mounting frame from the delivery (BOX) truck.

- Summary of job

Using 2 (two) Forklift trucks;

Remove or Load detector from a delivery (BOX) truck

- Affected systems

Transport (BOX) Truck

Forklift Trucks

High Bay area of test Lab Bldg. 58

High Bay door

North loading zone driveway

- List of PPE

Forklift truck seat belt

Gloves

Eye protection

Leather closed toe shoes

- Identify danger zones

Falling Load

Forklift proximities to truck

Forklift proximities to building

Load control

Pinch points

Crush zones

Surrounding area around Truck, High Bay door frame, and High Bay

- Number of people needed

6 (six)

a. 2 forklift operators

b. Load control guide (Person)

c. 2 Lookouts

d. Delivery truck driver

- List of materials

NONE

- Procedure

NOTE: Detector is transported on a rolling frame stand

Position (BACK) delivery truck into loading zone driveway

Allow ample room for fork lift movements to rear and one side of delivery truck

Lookouts will monitor area for unauthorized personal and keep them away

Roll detector frame toward rear of truck and chock wheels

Using one forklift truck support detector frame and lift off of truck loading deck

Secure frame to forklift using tie downs or equivalent means

Remove chocks from detector frame wheels

Follow hand signals from the Load Guide person

Slowly back detector frame out of truck

Expose enough of the frame for the second forklift to support the load from one side

Stop!

The Load Guide person will instruct the second forklift truck to pick up the detector

from the truck bed and supporting forklift

Once the load is fully supported; the truck driver will slowly drive away from the load

The other fork lift will move away from the load and clear the area.

The forklift supporting the Load will then be maneuvered to the High Bay area

The Load will be lowered to the floor and forklift truck will clear the detector frame

- Post job briefing and cleanup

Clear all equipment from area

Remove any tie downs and secure

- Document any problems encountered as “lessons learned”

- Procedure for loading detector

Reverse basic procedure to load detector into truck

Examine the task for hazards

Unauthorized personnel

Load proximities

Falling objects

Falling load

Pinch and crush points

Risk codes for hazards

Unauthorized personnel	1

Load proximities		2

Falling objects			2

Falling load			2

Pinch and crush points	2

Mitigate hazards

Lookout (person)

Load guide and use of hand signals

Tie downs

Rigging Training

Lookout (person)

Risk codes after mitigation

Unauthorized personnel	0

Load proximities		1

Falling objects			1

Falling load			1

Pinch and crush points	1