Right Dipole polarity reversal procedure/checklist

By Scot Spiegel and Mark Stevens

Revision:  January 23, 2001

Persons performing procedure: ________________________________________________date________

This procedure/checklist outlines the polarity reversal of the Right Dipole power supply main current output and the polarity reversal of all related equipment.  It is intended to be performed by qualified Jefferson Lab technicians who have been trained to perform it by the Hall A technical staff.

Note:

The Right Dipole is in (+)(hadron) polarity when the polarity reversal switch handle is in the up position, and in

(-)(electron) polarity when the polarity reversal switch handle is in the down position. The convention for all of the polarity switches/boxes is to match the positions of the devices (i.e., all handles/connectors in the up position = hadron polarity).

# Danger

The Right Dipole is a low-voltage, superconductive magnet system capable of storing lethal amounts of energy.  Because of an installed “freewheeling” or “persistent” switch, energy can be stored  even when the power supply is turned off.  You must be extremely careful to ensure that all energy has been extracted from the magnet system prior to moving the polarity reversal switch to a different position.  Failure to do so could result in death or severe injury to the person moving the switch.

- The Right Dipole Dynapower power supply, designated PD172, can only be operated in the remote control mode, by using the Hall A EPICS control software.  The power supply must be ramped down to 0 amps DC, +/- 1 amp DC as read from the DC CURRENT meter on the front of the Dynapower power supply.  If the magnet is at or near its maximum field, this step can take hours to complete.  Any attempt to expedite extracting the energy from this injured magnet (such as fast - dumping from high field) may result in further damage to the magnet. Verify the completion of step 1 by observing a reading of  000 or 001 amps in the DC CURRENT meter on the front of the Dynapower power supply, PD172.

Note

The normal operation of the M3 and M4 lamps on this Right Dynapower is different than the

Left Dynapower.  On the Right Dynapower, normally the M3 lamp is lit, indicating that

the power supply output is on.  When M3 is on, M4 must be off, and the system is operating.

When M3 is off, M4 will turn on, putting the system into a freewheeling mode, which very

slowly dumps the stored energy.  Some intentionally designed system failure modes can cause

both the M3 and M4 lamps to turn off, putting the system is in fast – dump mode.  The following

2 steps identify what mode the system is in, and artificially induces the desired failure mode.

2.____  Check that the M3 lamp is lit and the M4 lamp is extinguished on the front of the Dynapower power

supply, PD172.  If you observe any other indication of these lamps, the system is malfunctioning and you

must contact Mark Stevens before proceeding.

3.____  Then put the power supply into the fast - dump mode by performing a hard - reset of the lead voltage

controller:  In rack OD172Q locate the RIGHT DIPOLE DYNAPOWER INTERFACE CHASSIS, and push the single green button on the front of it.  You should see the two red LEAD VOLTAGE fault lamps light on the PROTECTION RELAY CHASSIS immediately above it.  You should hear a “clunk” sound from the Dynapower power supply as the M4 output contactor trips.  Verify this step by looking at the front of the Dynapower power supply and observe that both the green M3 and M4 lamps are extinguished.

Danger

The Dynapower power supply PD172 is supplied with uninterruptible 480VAC 3 phase

power and uninterruptible 120VAC single phase power.  Compliance with the

following steps mitigates these hazards.

4.____	Turn off the main input power switch on the front of the Dynapower power supply, PD172, a large lever-    type switch with a red and black handle on the right side of the front panel.

5.____	Turn off the power to the Dynapower at the safety disconnect switch marked PD172  DIPOLE MAGNET SAFETY DISCONNECT SWITCH, and lock it out in accordance with Jefferson Lab safety lockout procedures.

6._____	Unplug the 120VAC plug which provides uninterruptible power to the Dynapower.  This plug is located inside rack OD172Q, is plugged into a rack-mount uninterruptible power supply, and is marked by a CEBAF caution tag “CAUTION – Uninterruptible control power for the Right Dipole power supply – do not disconnect”.  Lock out the plug in a suitable plug lockout box in accordance with Jefferson Lab safety lockout procedures.  Verify that no status/annunciator lights are lit on the front of the Dynapower.

7.____	Place a clamp-type DC amp meter on the large (535 MCM) main current lead between the Dynapower and the magnet in the location described in the “Danger” note above.  Ensure that the jaws of the clamp are completely closed and the meter is turned on to its lowest range. Verify a reading of 0.0 amps DC on the clamp-type DC amp meter.

#### Danger

### Place the clamp-type DC amp meter on one of the main current carrying cables between the Dynapower polarity reversal switch and the magnet, not between the Dynapower and the dump resistor cabinet.  The correct cable can be found in the cable tray marked “>>TO MAGNET>>”.  It is wrapped with red tape and on the red tape appear the words “PUT CLAMP METER HERE”.

8.____	With the clamp meter in place, make a test circuit with a short piece of at least 22 awg wire and a D-cell flashlight battery.  Pass a loop of wire through the amp meter and connect it to the flashlight battery.  You should be able to see a current of several amps DC with this test setup.  After verifying the operation of the amp meter, again observe a reading of 0.0 amps DC.

9.____Verify a reading of (either + or - )0.010 VDC or less on the QUENCH PROTECTION CONTROLLER,

located in rack OD172Q.  This meter reads the voltage across the main current leads at the magnet.  With the system in the fast - dump mode, this meter reads the voltage drop across the fast - dump resistor:

Any significant difference of potential means that a significant current is still flowing.  If the voltage is higher than 10mVDC, and the voltage is decreasing, the magnet is still discharging.  You must wait until the voltage is 10mVDC or less!

10.____	Unlock and open the polarity reversal switch door.  Place a DC voltmeter voltmeter (set to read milli-volts, typically the 300mV switch position) across the center contacts of the switch (across the moving handle pivot points).  In this step you are reading the voltage drop (if any) across the dump resistor from any current which may be flowing through it.

Any significant difference of potential means that a significant current is still flowing.  Observe a reading of 10mVDC or less.  This voltage should be approximately the same as step 10.  The polarity of the voltage is a function of the polarity of the magnet system, and may be either positive or negative (the polarity is irrelevant-what is important is the amplitude). If the voltage is higher than 10mV, and the voltage is decreasing, the magnet is still discharging.  You must wait until the voltage is 10mVDC or less!

11.____ Verify the proper operation of the DC voltmeter by measuring a reference source of 10mVDC

provided in the polarity reversal tool box.  Turn on the DVC-350A calibrator.  Ensure that

the DEC/HEX switch is in the DEC position.  Ensure that the 1.2v-DEC-12v/1.0v-HEX-10v switch is in the right-most switch position.  Ensure the polarity switch is in the “+” position.

Push the “left arrow” button once.  Push the “up arrow” button once.  A reading of  “+0.010” should be displayed.  Place the DC voltmeter meter lead probes into the “output” sockets on the side of the calibrator.  The meter should read 10mV, plus or minus a few millivolts.  If the meter fails this test, repeat steps 10 and 11 with another meter until both steps are successfully completed.  If the meter fails this test, repeat steps 10 through 12 with another meter until all steps are successfully completed.

12.____	If all of the previous steps have been successfully completed, the magnet has been sufficiently discharged, and is now safe for polarity reversal by following the next step.

13.____	Using at least 18 gauge wire (a meter-lead type wire is fine) and alligator clips, short across polarity reversal switch from pivot point to pivot point.  This is a safety device which provides a path for several amps of current if for any reason all of the previous precautions overlooked 10 amps or less of current in the magnet.  With the shorting jumper in place, move the polarity reversal switch to the new position.  Remove the shorting jumper.  Close and lock the switch box.

14.____	Change the plugs on the current limiting resistor box to the correct box for the polarity you are changing to.  The boxes are marked (HADRON) POLARITY and (ELECTRON) POLARITY.  Ensure both cables, labeled P1 and P2, are plugged into the same box.

15.____	Change the polarity of the Field Gradient Compensating Coils for the Dipole NMR probe by first turning off the Kepco power supply in rack 1H72B06 and then moving the Field Gradient Compensating Coil Polarity Reversal Switch, located in the same rack, to the appropriate position  Make sure the Kepco is turned back on!

16.____	Change the polarity of the Dynapower remote voltage sense.  On the front of the Dynapower power supply, locate the remote voltage sense polarity reversal switch.  It is marked “(+) (HADRON) POLARITY” and “(-)(ELECTRON) POLARITY”.  Place the switch in the desired position.

17.____	Unlock and restore 120VAC power to the Dynapower power supply.

18.____	Unlock and restore 480VAC power to the Dynapower power supply.  Turn on lever-type switch on the power supply.

19.____	Using Hall A EPICS software, reset faults and test for proper operation.