1. Define the scope of work
	Replace all socketed chips on Q1 power supply Programmable Ramp Board.

2.  Analyze Hazard

| Task | Potential Hazards | Risk Code before mitigation | Safe Procedures/Practices/Controls | Risk Code after mitigation |
| --- | --- | --- | --- | --- |
| Replace all socketed chips on programmable ramp board. | Low voltage electrical shock, chip damage | 0 | Turn off power before changing chips | 0 |

3. Develop and implement Hazard controls

Turn off power before changing chips

4. Perform work within controls (follow procedure)

- Summary of job

  - Replace socketed chips on Q1 power supply Programmable Ramp Board.

- List of PPE

  - None required

- Identify danger zones

  - An area extending 36 inches out from the Q1 Danfysik power supply control closet door,  when power is applied to the power supply.

- Number of people needed

  - 1 electrical/electronic or mechanical tech and qualified electrical worker, with the following minimal training:  Rad Worker I; Jlab Lock, Tag and Try;  Hall A Safety Walkthrough

- List of materials

  - The following chips:  AD558JN, qty.1;  TLO72CP, qty. 3.

- Procedure

  - Observe the power supply control panel.  If the “Main Power” lamp is green, observe the display panel – “SET” should be all zeros.  If it is not zeros, go to the Controls Interface Terminal next to the green steel block wall and select the Q1 power supply gui from the “Hall A General Tools” gui.  Type in zeros into the “Setpoint” box, and press enter.  Go back to the power supply control panel display.  The “SET” and “ADC” (which reads in per cent – with .999999999 being the maximum) should read zeros, but a small amount of offset is normal in the “ADC” reading.

- Once the “ADC” reading is zeros, push the “local/remote” button to extinguish the remote control lamp.  The “Main Power” lamp should extinguish; if it does not, push the “Main Power Off” button.

- Turn off power at the safety disconnect switch marked PQ17(1 or 2).

- Open the control panel door.

- Locate the Programmable Ramp Board, part no. 83140.

- Replace the following chips:  AD558JN, qty.1;  TLO72CP, qty. 3.

- Close and lock the control panel door.

- Restore power to the power supply.

- Test power supply for normal operation.

- Post job briefing and cleanup

5. Provide feedback and continuous improvement. (Document any mistakes, corrections, changes and pictures and implement changes.)

Directions: Save this template as a new file name that contains the title. Double click on the header and update the title, author and date, the pages are automatic. Double click outside the header and fill in the bulleted information by following the instructions in italics. When you are finished with the document delete everything in italics. Send the completed document to the Hall coordinator for review and approval.

1. Define the scope of work

- Job Summery

- Affected systems

Describe the job in a step by step procedure using an outline format that someone else trained in this area of expertise could understand and use to do the job. If a task is outside of normal daily operations and standard industry standards a task Hazard analysis form is required

2. Analyze Hazard

- Place THA here

Examine the task for hazards with the Task Hazard analysis worksheet

Look at all aspects of job and its associated hazards.

Perform a pre-job walk down

Assign a risk code for each hazard before mitigation

Hazard analysis and risk code information can be found in section 3210 of the ESH&Q manual

3. Develop and implement Hazard controls

- List of PPE

- Identify danger zones

- Training required

Explain how risk(s) is/are mitigated

Assign a risk code for each hazard after mitigation

Write work procedure and incorporate hazard mitigation.

4. Perform work within controls

- Number of people needed

- Verify  that needed training is completed and current

- List of materials

- Procedure

Perform pre-job briefing (1st step)

- Post job briefing and cleanup

Follow procedure

5. Provide feedback and continuous improvement.

Document any mistakes, corrections, changes and pictures and implement changes. Turn in feedback to Hall work coordinator

Example

Big Bite Electron Detector Frame Modification

1. Define the scope of work

- Job Summery

This description of work will explain the modification of Big Bite Detector frame. Using the overhead crane to remove sections of frame and add tube extensions and 4” tube uprights, welding in place.   Cut and remove 4” square tube corner supports on frame. Grind welds down flat at ends of tube.

Weld tube extensions to frame and weld new support uprights.

- Affected systems

Overhead crane

High bay area of test Lab Bldg. 58

Big Bite detector stack frame

Transversity Equipment test area

Detector frame hardware

2. Analyze Hazard

Task Hazard Analysis Worksheet

A-08-067-T

| Complete all information. Use as many sheets as necessary. |  |
| --- | --- |
| Task location test lab | Task title BB electron detector frame Mod |
| Division Physics | Prepared by Folts |
| Department Hall A | Reviewed by Dezern |
| Supervisor deJager | Approved by Folts |
| Standard Requirements: (PPE, TOSPs, SOPs, etc.) |  |

| Task | Potential Hazards | Risk Code before mitigation | Safe Procedures/Practices/Controls | Risk Code after mitigation |
| --- | --- | --- | --- | --- |
| Modify BB electron detector frame | Crane Crush, Bump, Pinch Drop load Cutting, grinding, welding Fall | 3 3 2 | Correct rigging, Minimum elevation Clear area of personnel and equipment Use steel toed shoes and safety glasses Use warning beacons and guards when appropriate Use hard hats when deemed necessary Equipment specific training Proper PPE, Crane training Hot work permit, safety glasses, face shield, fire watch, fire extinguisher Ladder training, harness training and a harness when necessary | 1 1 0 |

3. Develop and implement Hazard controls

- List of PPE

Hard Hat

Gloves

Eye protection

Safety Shoes

Welding Shield

- Identify danger zones

Falling Load / objects

Load control

Pinch points

Crush zones

Local areas within lift boundaries

- Training required

Welding

Fire watch

Ladder

Crane

Power hand tools

Equipment specific training

4. Perform work within controls

- Number of people needed

1-person to rig and fit, 1-crane operator, 1 fire watch

- Verify  that needed training is completed and current

- List of materials

Slings

Edge protectors (as needed)

Shackles

Frame extensions and, 4”s.s.tube

Drawings

- Procedure

Pre job briefing and or job walk down

Warning cones and welding screens set in place

The next 4 steps need to be done one corner at a time in order to maintain the integrity and alignment of the structure

Install temporary support prior to cutting corner support

Using port-a band or sawsall cut out 4” corner tube

Cut and grind welds down for clearance of new parts

Using the crane, position and clamp the new vertical support and weld as per the supplied drawings.

Prepare the ends of the horizontal beams for installation of extensions by removing paint and prepping metal for welds.

Align new horizontal extensions and weld as per the supplied drawings

- Post job briefing and cleanup

Gather any loose hardware and store in proper location

Secure slings, shackles

Clear all equipment from area

5. Provide feedback and continuous improvement.

Document any mistakes, corrections, changes and pictures and implement changes. Turn in feedback to Hall work coordinator