1. Define the scope of work

- Job Summery

The polarized 3He hemholtz coil assembly is use for the polarized 3 He family of experiments. It consists of an aluminum base holding 2 pairs of coils in the vertical orientation with the pairs being perpendicular to each other. Each coil weighs approximately 250 lbs. the base weighs 500 lbs bringing the total without any of the inner parts to1500 lbs. The assembly is rigged with a choke through the center whole and around the base of each of the coils. The slings are then routed through the center of the upper coil support to the hook. The assembly is lifted over the cryo lines and utilities and into position over its base that will already be bolted to the center post and lowered onto the post as per this instruction.

- Affected systems

Hall A pivot area

Crane

2. Analyze Hazard

Task Hazard Analysis Worksheet

A-08-109-T

| Complete all information. Use as many sheets as necessary. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task location Hall A |  |  | Task title Remove/install target coils |  |  |
| Division Physics |  |  | Prepared by Folts |  |  |
| Department Hall A |  |  | Reviewed by |  |  |
| Supervisor deJager |  |  | Approved by Folts |  |  |
| Standard Requirements: (PPE, TOSPs, SOPs, etc.) |  |  |  |  |  |
| Task | Potential Hazards | Risk Code before mitigation |  | Safe Procedures/Practices/Controls | Risk Code after mitigation |
| Remove Cryo target chamber Elevated work Crane operations | Radiation dose Radiation contamination Fall Drop tools Back strain Bump, pinch Dropped load | 3 3 3 |  | RWP, RAD II training Use if proper equipment, ladder training, manlift training, harness training, keep area under work clear of personnel Equipment training Proper PPE, Crane training | 1 1 1 |

3. Develop and implement Hazard controls

- List of PPE

Hard Hat

Safety shoes

Gloves

Harness if necessary

- Identify danger zones

Area around the target  balcony

- Training required

Hall A safety training

Rigger training

Cranes training

4. Perform work within controls

- Number of people needed

1 rigger/observer

1 crane operator

- Verify  that needed training is completed and current

- List of materials

Slings and rigging hardware

Dunage

4 screw jacks

4 large C clamps

- Procedure

Perform pre-job briefing by reviewing this document

4 ½ inch swivel lifting eyes are installed into the inner holes in the support plate and torgued to 28 ft lbs

4 12 ft slings are attached to the lifting eyes and the hook

The assembly is lifted over the cryo lines and utilities and into position over its base that will already be bolted to the center post and lowered onto the post as per this instruction.

- Post job briefing and cleanup

5. Provide feedback and continuous improvement.

Document any mistakes, corrections, changes and pictures and implement changes. Turn in feedback to Hall work coordinator