The HRS-R dipole field is continuously drifting down. The current control panel shows "Coil Voltage: Quench". Looks like a problem...

Ed (tech on call) confirms that the HRSR dipole is in protection mode and ramping down very slowly. Until it reaches near zero current, there is nothing we can do to restart it. Recovery will probably take several hours. Bad news for the right arm data...

HRSR dipole fault cleared. Current is at zero. Restarting magnet...

HRSR dipole is behaving strangely. After reset and restart, it began to ramp the current up, but the rate of ramping became slower and slower and finally zero around 26A. Then it began to ramp back down at an extremely slow rate (maybe 5mA/sec). The system seems stuck in this mode. I'll let it settle down to zero by itself for now. An expert should look at it at the next opportunity.

I gave it one more try. 
Clicked "on"……. power supply fault
Clicked "reset"….. fault cleared, power supply says "starting", then "on" and "engaged" (in red)
Entered current set point of 100…..coil voltage says "quench"
A few seconds later, the power supply does "off" and "disengaged" (green). Current ramps up very slowly. Rate of ramping decreases.  I give up on this one.

After another "reset", "on", "current set" cycle, the HRSR dipole current is now actually ramping with power "on" and "engaged" (red). Ramping rate of somewhere near 200 ma/sec.