Rebooting Spectrometer IOC's etc. 

left(right) 

go to hareboot10(25) [username:hlauser etc.] 
select iocha14 (iocha16) 
select "immediate reboot" 
click "next" 
click "apply" 
Wait it takes a few minutes 

Rebooting NMR's etc 
go to hareboot17(27) 
select the system you want to reset [nmr's and/or hall's] 
select "immediate reboot" 
click "next" 
click "apply" 
verify that the reset sytem is alive and responding on the controls screens. 
[If they aren't you may need to go back and select "Immediate On" see halog copied below] 

After any of the above reboots check in the NMR controls screen 
[it's in the fields pull down menu on the tools screen] make sure: 
"search" is ON 
"regulator" is ON 
[if things still don't seem right i.e. acting like it's OFF when it says ON 
turn them OFF and then back ON again] 

Here you may need to reenter your set P0 on the tools screen. 
Remember the controls won't recognize the new number unless it's 
different from the last. If it's already at the correct value, enter a 
slightly different value and then enter the correct value. 


Log entry time 15:50:04 on December 01, 2006 
Entry number 189671 
keyword=Rebooting Right NMR 

When you reboot the right dipole nmr it isn't enough to reboot. You must 
also turn it back on. 
Proceedure: 
go to hareboot27 
select port 8 (labeled "Right Dipole NMR") 
select control action "Reboot immediate" 
click "next" 
then click "apply" 
after that is done (reloading the hareboot27 screen will show that the 
NMR is ON. It isn't) 
again select port 8 
select control action "On Immediate" 
click "next" 
then click "apply" 
NOW you should be OK 

NOTE: 
The search setting on the NMR GUI has a quark worth mentioning.  If it is displaying "off", and will not respond to an "on" command, click off, and then on.