Brad paged me with the original problem that the RQ3 hall probe was

not reading back correctly.  After having to drive in to do anything

(firewalls are so great) RQ3 came up to momentum.  Then we noticed

the dipole had dumped.  Well, ramped down.  By the end of fooling

around, the dipole consistently shows an FC Fault and a ZFCT Fault.

The EPICS screen shows no faults, all green.  One could say it was a

problem with the octagon, but at one point it started to ramp up all

on its own.  I'm assuming it was going to the P0 setting.  I hit

"0 AMPS" and it ramped down from there with no response to anything

we tried.  So, I kind of doubt it is the octagon, since it tried to

ramp the supply at one time, prior to a time when I know the faults

would not clear.